Placing Bar Code CID Labels on Transfusion Service Specimens

| ##### Purpose: | The purpose of this procedure is to provide instructions on how to add a CID bar code label onto a Transfusion Service labeled specimen. |
| --- | --- |

| ##### Policy: | - Specimens must be evaluated for acceptability before use. See Determining Specimen Acceptability for Transfusion Service Testing procedure. - All specimens that are run on the automated analyzer must have a CID bar code label attached for identification by the analyzer. - If samples are received without a Clinical Collect bar code label, the SQ bar code label will be applied by the blood bank technologist after verifying that the sample meets specimen identification criteria. - If SQ bar code label is applied in the Transfusion Service, the patient identifiers on the original label including blood bank armband number of the specimen must be legible. - All bar code labels must be marked with an identifier of the person who applied the label. - Note: Non-pretansfusion samples that are received with other patient identification labels will have CID labels applied by the Transfusions Service Technologist as specified in the previous statement. |
| --- | --- |

| ##### Procedure: | ______________________________________________________________ |
| --- | --- |

| Step | Action |  |  |  |
| --- | --- | --- | --- | --- |
| 1. |  |  |  |  |
|  |  | If: | Then: |  |
|  |  | Specimen has a Clinical Collect bar code attached that corresponds to test ordered | This procedure is not needed |  |
|  |  | Specimen meets acceptability criteria but does not have a CID bar code label attached that corresponds to test accession number | - Locate or print the SQ CID label for the specimen - Verify that the patient information on the sample matches the patient identifiers on the CID label - Add initials/ code or other identifier accepting responsibility for relabeling of sample tube. - Proceed to step 2. |  |
| 2. | Add bar code label vertically on Blood Bank armbanded specimen below the DOB being careful that the name, medical record number, DOB and BB armband number are still legible. |  |  |  |
| 3. |  |  |  |  |
|  |  | If: | Then: |  |
|  |  | Bar code fits on tube so that BB armband number is visible | Procedure is complete |  |
|  |  | When adding bar code, the BB armband number is partially obscured | Add another BB armband number to the tube. |  |