#### Performing a Low Ionic Strength (LISS) Antibody Screen/Panel

| ##### Purpose | This procedure describes how to perform a Low Ionic Strength (LISS) test method in order to detect unexpected blood group antibodies. |
| --- | --- |

| ##### Policy | This procedure is used: - when troubleshooting routine compatibility test methods - As part of an antibody identification workup, when indicated. |
| --- | --- |

| ##### Quality Control | Test screening cells with additive against QC serum and Monoclonal control each day manual antibody screen is used for patient testing. Record on QC form. |
| --- | --- |

| ##### Equipment / Reagents & Supplies |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Equipment | Reagents | Supplies |  |
|  |  | - Centrifuge - Heat block (36-38C) - Optical aid - Serological cell washer | - 3-5% reagent red blood cells - LISS Additive (ORTHO Antibody Enhancement Solution) - Anti-IgG human Globulin - Coombs control cells - 0.9% Buffered BB Saline | - Test tubes - Test tube rack - Plastic pipettes |  |

| Specimen Requirements | One 9 ml EDTA stoppered tube is preferable. Serum collected from a red stoppered tube without separator gel is also acceptable. |
| --- | --- |

| Procedure | Follow the steps below to perform this procedure: |
| --- | --- |

| Step | Action |  |
| --- | --- | --- |
|  | - For antibody screening or identification, reagent red blood cells may be used directly from the vial. |  |
|  | Label each tube with a patient and cell identifier. |  |
|  | Add the following to each tube: - 2 drops of patient plasma or serum - 1 drop of the appropriate cell suspension - 2 drops of Antibody Enhancement Solution |  |
|  | Mix well and incubate all tubes at 37oC for 15 minutes (maximum of 30 minutes). |  |
|  | Centrifuge for time posted on centrifuge. Examine the supernatant for hemolysis. |  |
|  | Resuspend the cells by gentle agitation. |  |
|  | Examine macroscopically for agglutination. Grade and record results. |  |
|  | Wash the cells 3-4 times with 0.9% saline with serological cell washer or manually decanting to a dry cell button. |  |
|  | Add 2 drops of Anti-IgG to the dry cell button and mix well. |  |
|  | Centrifuge for time posted on centrifuge. |  |
|  | Resuspend the cells by gentle agitation |  |
|  | Examine macroscopically using an optical aid (magnifying glass) to verify final reading. Note: Microscopic examination is discouraged when using this reagent. |  |
|  | Grade and record results. |  |
|  | Add 1 drop Coombs Control cells to each negative reaction. |  |
|  | Mix and centrifuge for time posted on centrifuge. |  |
|  | Resuspend the cells by gentle agitation. Grade and record results. |  |
|  | If: | Then: |
|  | No agglutination in the IgG coated coombs control cells. | The test must be repeated. |
|  | Macroscopic agglutination in the IgG coated coombs control cells. | Indicates that the anti-human globulin added was capable of reacting. |

| Method Limitations | - Equal volumes of plasma/serum and additive are required in order to achieve desired final ionic strength. - Weaker reactions may be obtained if tests are incubated less than 10 or greater than 30 minutes. - Complement dependent antibodies may not be detected if plasma is used. |
| --- | --- |

| Interpretation | Reference Range: Negative |
| --- | --- |

| If: | Then: |
| --- | --- |
| No agglutination or hemolysis is present. | Result is negative - Indicates the absence of an antigen/antibody reaction. |
| Hemolysis and/or agglutination are present. | Result is positive. - Indicates the presence of an antigen/antibody reaction. - Refer to Identifying an Antibody Procedure |

| Reporting Results | Insert Testing grid by adding test ASL or using L on the keyboard N=Neg P=Pos |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Antibody Screen LISS |  |  |  | Interp |
|  |  |  | 37C INC | AHG | CC | N |
|  |  | S1 | 0 | 0 | 1-4 |  |
|  |  | S2 | 0 | 0 | 1-4 |  |
|  |  | S3 | 0 | 0 | 1-4 |  |

|  | 37C INC | AHG | CC | Interp |
| --- | --- | --- | --- | --- |
| S1 | 0-4+ | 0-4+ | ND-4+ | Any cell S1-S3 Pos at 37C INC or AHG P |
| S2 | 0-4+ | 0-4+ | ND-4+ |  |
| S3 | 0-4+ | 0-4+ | ND-4+ |  |

| References | - ORTHO Antibody Enhancement Solution Package Insert e631206614_EN |
| --- | --- |

| Related Documents | - Interpreting an Antibody Identification Panel. - Grading Serological Tube Reactions |
| --- | --- |