I.  	Purpose:

To define the Soft computer process when resulting Special Coagulation QC.

II.  	Procedure:

Controls

MCOAG1 – Citrol 1

MCOAGP – Control Plasma P

MCOAGN – Control Plasma N

How to Order Factor Assays and RCF QC:

- Click on Orders

- Click on Order Entry

- Click on

- Item Type: Control

- Item ID: MCOAGP

- Click on Find

- Uncheck any tests that do not have QC being run

-Example: Need to order QC for Factor VIII and Factor IX.  Only tests F810 and F910 should be checkmarked for Workstation MBCS3.

- Click

- Click Yes to “Do you want to save changes?”

- Order Entry will bridge to Result Entry

- Close Result Entry tab

- Click on Order Entry tab

- Click on

- Item Type: Control

- Item ID: MCOAGN

- Click on Find

- Uncheck any tests that do not have QC being run

-Example: Need to order QC for Factor VIII and Factor IX.  Only tests F820 and F920 should be checkmarked for Workstation MBCS3.

- Click

- Click Yes to “Do you want to save changes?”

- Order Entry will bridge to Result Entry

- Close Result Entry tab

How to Run and Result Factor Assays and RCF QC:

- Order QC tests in BCS for controls to be run on

- Go into Instrument menu

- Highlight Order that contains the Lot Record MCOAGP

- Click

- If any results are out the screen will bridge to TQC

- Choose Action from Action ID then Click OK

- When all results are posted and popup will appear stating “test(s) were posted to QC order #”

- If QC is out a new QC order and label will be generated depending on the Action taken (A new order will be generated and linked to the original QC order)

- Click OK (the label is not needed but will automatically be printed)

- Rerun controls and repeat above steps if needed

- Highlight Order that contains the Lot Record MCOAGN

- Click

- If any results are out the screen will bridge to TQC

- Choose Action from Action ID then Click OK

- When all results are posted and popup will appear stating “test(s) were posted to QC order #”

- If QC is out a new QC order and label will be generated depending on the Action taken (A new order will be generated and linked to the original QC order)

- Click OK (the label is not needed but will automatically be printed)

- Rerun controls and repeat above steps if needed

- When all controls are in patient samples may be run

How to Order Thrombin Time QC:

- Click on Orders

- Click on Order Entry

- Click on

- Item Type: Control

- Item ID: MCOAG1

- Click on Find

- Uncheck all tests except for TT

-Example: Only tests TT should be checked for Workstation MBCS3.

- Click

- Click Yes to “Do you want to save changes?”

- Order Entry will bridge to Result Entry

- Close the Result Entry tab

How to Run and Result Thrombin Time QC:

- Order QC tests in BCS for TT

- Go into Instrument menu

- Highlight Order that contains the Lot Record MCOAG1

- Click

- If TT result is out the screen will bridge to TQC

- Choose Action from Action ID then Click OK

- When TT result is posted and popup will appear stating “1 test(s) were posted to QC order #”

- If QC is out a new QC order and label will be generated depending on the Action taken (A new order will be generated and linked to the original QC order)

- Click OK (the label is not needed but will automatically be printed)

- Rerun control and repeat above steps if needed

- When TT controls is in patient samples may be run

III.	HISTORY:

H-1	This procedure was written by Jessica Kaul on 3/7/2010.

H-2	This procedure was revised by Jessica Kaul on 7/20/2010.