I.	Purpose:

In the special coagulation laboratory, the results the test listed below must be manually entered into the Soft computer system:

- FXIII

- Inhibitor titers

- Mixing studies

Prot S Total and Free antigen

Accuracy of manual entry is critical to the timely reporting of results to a patient’s caregiver, and it is equally important to document that accuracy.

- II.	Procedure:

1.   Determine what tests are to be run, by calling appropriate pending lists.

- For the STA Compact, scan the barcode label to put the sample on the instrument (this is a change from our current process of typing the patient name)

- For the BCS, place specimens on the instrument with barcode labels, as usual.

- Run the tests as usual; go to Instrument Menu and open Stago1, Stago2, or BCS3 as appropriate, and post the results (for those tests which can be posted)

- For the tests listed above, or for any other results which cannot be posted, make a copy of the instrument printout.

- Attach the patient’s barcode label, or the entire pending list to the instrument printout.

- Manually enter the results using the instrument printout.

- When all manual results for that test have been entered, call a Verified Result Report as follows:

  - On Soft Lab Mic Live, click on Reports on the tool bar

  - From the drop-down list click on Verified Result Report

  - Put in the date range of your pending list (eg: 8/1/2011 to 8/2/2011)

  - Unclick all the comment boxes

  - Location should be ALL; Department EMPTY; and Workstation EMPTY.

  - Highlight the Test ID box (near the bottom) and put in the test(s) needed

  - Make the printer H15 Laser Sp Heme if necessary

  - Click Print to box and allow Soft to create the report. (Report will print automatically, but may take a minute to create.)

- Additional information for specific tests:

  - P2Y12

    - Type order # as patient ID

    - Print out each result and write patient last name on it

    - Enter results into Soft from the printout

    - Call Verified Result Report and attach the printouts to it

    - Leave for supervisory review

  - Low factor assays (or those which for any reason do not autoverify); mixing studies; Low RCFs (or those which for any reason do not autoverify)

    - Print out patient’s result(s) from BCS Job List

    - Enter results from the printout

    - Print a label for that sample and place it on the printout

    - Print a Verified Result Report and attach to the BCS printout

    - Leave for supervisory review

  - FXIII

    - Call a pending list for FXIII

    - Place label on the worksheet in the FXIII notebook

    - Run test and enter result in usual way

    - Call Verified Result Report and attach to pending list

    - Leave for supervisory review

  - Any other STA Compact or BCS test results which cannot be posted

    - Enter the result manually from the instrument printout

    - Print out the patient result

    - Print a label for that sample and place it on the printout

    - Print a Verified Result Report for that test or group of tests

    - Attach all together and leave for supervisory review

- Attach the Verified Result Report to the other paperwork and leave on supervisor’s desk for review.

- Supervisor will tally and fill out the monthly log for transcription errors (Sec 10 in notebook) before discarding verified reports.

- If any transcription errors have occurred, supervisor will document action taken to remedy (speak to tech responsible for the error, assure that patient report is appropriately amended and follow-up notification documented).

- III.	History:

H-1	This procedure was written by N Kriz 8/5/2011.

H-2	This procedure was revised by N Kriz 6/26/2012.

H-3	This procedure was revised by N Kriz 6/12/2013.