I.  	Purpose:

To define the Soft computer process when resulting from the ESR template.

II.  	Procedure:

Orderable Tests:

BSINT = Blood Smear Interpretation

ESR = Sedimentation Rate (ESR)

RETIC = Reticulocyte Count

PMALA = Malaria Detection and Quantitation

PBABE = Babesia Detection and Quantitation

PEHRL = Ehrlichia Smear

ESR (Day shift) Resulting Template: for ESR, RETIC, BSINT and Parasites

MESRE (Off shifts) ESR Resulting Template: for ESR, RETIC, BSINT and Parasites (main lab, Smilow lab, and North Haven lab locations)

A.  How to check and print pending results:

- Open Resulting Worklist:  template - ESR

status -  Pend and Nonver

from order # - current orders

- Any orders on the resulting worklist have either an ESR, RETIC, BSINT or Parasite pending

- Click on the printer icon and Click on Worklist

- A Print Worklist box will appear

-Choose the correct printer to print to by clicking on the ▽ and then

clicking on the printer name

-Leave the default layout as Horizontal

-Click OK

B.  To Verify ESR Results:

- Results will auto-verified with the exceptions of results < 5, >120, with error flags, and samples not collected and received.

    - Results >20 will flag AB (abnormal) and have a yellow box

    - Results that are >120 from the instrument will appear automatically as >120 in instrument menu.

- For samples not auto-verified, follow the ESR-Auto Plus procedure for checking the ESR sample tube prior to verifying the result

    - Open Instrument Menu, Select Instrument MESR1 (ESR instrument #1) or MESR2 (ESR instrument #2), Click OK.

    - Select order number to be verified and Click Post All on the Instrument Tab

C.  To Verify Parasites:

- Open Resulting Worklist:  template – ESR

- status -  Pend and Nonver

- from order # - current orders

- Select order # for results to be entered

1.  Resulting a PMALA

- In resulting worklist Test PMAL will always be required.

  - Select positive or negative from the keypad

- If result is negative

  - Tests MQUAN and BINAX will already have "." in the resulting field, then simply click Verify all when resulting PMAL.

- If result is positive

  - Needs Quantitation enter the % in MQUAN and Click Verify & Save

  - Needs Binax select the appropriate interpretation comment in BINAX from the keypad then Click Verify & Save

    - @T1P	Positive result for P. falciparum

    - @T2P	Positive result for P. vivax or P. malariae or P. ovale

    - @T12	Positive result for P. falciparum, possibly a mixed  infection of P. falciparum with another species

    - @NO12 Negative – no malaria antigens were detected

- @INV	Invalid and/or uninterpretable results

- If both MQUAN and BINAX needs to be resulted enter results and click verify all

2.  Resulting a PBABE

  - In resulting worklist Test PBAB will always be required.

  - Select positive or negative from the keypad

  - If result is negative

    - Tests BQUAN and BINAX will already have "." in the resulting field, then simply click Verify all when resulting PBAB.

- If result is positive

  - needs Quantitation enter the % in BQUAN and Click Verify & Save

  - needs Binax select the appropriate interpretation comment in BINAX from the keypad then Click Verify & Save

    - @T1P	Positive result for P. falciparum

    - @T2P	Positive result for P. vivax or P. malariae or P. ovale

    - @T12	Positive result for P. falciparum, possibly a mixed  infection of P. falciparum with another species

    - @NO12 Negative – no malaria antigens were detected

- @INV	Invalid and/or uninterpretable results

- If both BQUAN and BINAX needs to be resulted enter results and click verify all

3.  Resulting Ehrlichia

- Enter results using the keypad

- Click N for Negative or Click P for Positive on the Key Pad

- Click Verify and save to verify the order

III.	HISTORY:

H-1	This procedure was written by Jessica Kaul on 3/7/2010.

H-2	This procedure was revised by Jessica Kaul on 3/8/2011.

H-3	This procedure was revised by Elizabeth Degano 7/19/2013.