I.	PRINCIPLE OF TEST:

Serial three-fold dilutions of patient plasma are incubated with normal pooled plasma for 	two hours. After this time, each dilution is assayed for residual Factor VIII activity. The 	reciprocal of the dilution inhibiting 50% Factor VIII activity is referred to as the inhibitor 	titer and the result is expressed in Bethesda units (B.U.s).

NOTE: The inhibitor titer will be affected by the presence of transfused Factor VIII 	products. Therefore, an inhibitor titer will NOT be done if the patient’s Factor 	VIII 	level is more than 5% above his baseline (untreated) level. (As examples, if the 	patient’s baseline is 0% FVIII, the 	titer will not be done if the current FVIII level is 	>5% activity; if the patient’s baseline FVIII is 5%, the titer will not be done if 	the 	current FVIII level is >10% activity.)

II.	REAGENTS:

All reagents are the same as for the Factor VIII assay

**Use in-house laboratory-pooled normal plasma **

##### III.	PROCEDURE:

A. 	Prepare three-fold dilutions of patient plasmas as follows

1.	1:1	undiluted

2.	1:3	0.2 ml of undiluted + 0.4 ml IBS

3.	1:9	0.2 ml of 1:3 dil'n + 0.4 ml IBS

4.	1:27	0.2 ml of 1:9 dil'n + 0.4 ml IBS

Etc. as needed

IV.	NOTE:

A.	These serial dilutions should be carried out to encompass the HIGHEST titer that 	might be encountered. This prediction might be facilitated by reference to the 	patient's mixing study, and to the patient's earlier inhibitor titers when these are 	available.

B.	Add 200 ul (0.2 mL) of each of the above dilutions to 200 ul (0.2 mL) of normal pooled plasma in 			BCS sample cups. Cover with plastic caps and incubate for 2 hours at 37oC.

C. 	As a control, add 200 ul (0.2 mL) IBS to 200 ul (0.2 mL) normal pooled plasma and incubate for the 			same period of time.

D.	Run Factor VIII determinations on the BCSXP according to procedure.

E. 	Determine the % Factor VIII activity for patient and control samples by reference to 			the standard curve, in the usual manner.

F.	Select the patient dilution that inhibits approximately 50% of Factor VIII activity, as 			compared with the incubated control. Determine the residual Factor VIII activity as 			follows:

% 	Patient Fac. VIII activity

------------------------------          x 100  =   % Residual Fac.VIII

% Control Fac. VIII activity                    	activity

G. 	Express results as Bethesda units by reference to the graph below, being sure to 			multiply the units times the dilution used, to obtain the final Bethesda units. If result 		is <0.25 BU, report “0” Bethesda Units

; if  result is  0.25-1 BU,  report the actual 			number Bethesda Units;  for results >1 BU, use only whole numbers not decimal points.

H.         	One "Bethesda Unit" is designated to be an inhibitor unit which neutralizes 0.5 units 			of Factor VIII activity in vitro, under the standard conditions described in this 	procedure. This does not indicate that any particular number of units of Factor VIII 	infused into a patient will neutralize a specific number of inhibitor units.

(REF: Coleman, R.W. et al.  Hemostasis and Thrombosis, pp 566-567.)  J.B. 				Lippincott Co., 1982.

V.

HISTORY:

H-1	This procedure was written by N. Kriz on 12-22-2004.

H-2	This procedure was revised by N. Kriz on 2-1-2010.

H-3	This procedure was reviewed by N Kriz on 2/3/2011

H-4	This procedure was reviewed by N Kriz on 12/1/2011

H-5	This procedure was revised by N Kriz on 12/19/2012.

H-6	This procedure was revised by N Kriz on 12/30/2013.