- Purpose:

To define for all hematology laboratory staff when and how to perform a smear scan.

- Policy:

The Laboratory policy for Manual Differentials states that the Laboratory will determine whether a patient receives a manual differential or a smear scan based on specific flags and data levels which need validation. If, during a scan review clinically relevant data is deemed to need further validation a manual differential is added and performed. The addition of a manual differential is consistent with the laboratory’s reflex test policy.

- Procedure:

- Specimen fails autoverification by either patient data levels and/or instrument flags

- Valid data is verified and released

- Patient instrument results print-out is generated

- Result(s) to be validated are noted on print-out

- Smear is made and stained (following smear and stain procedures)

- Print-out and slide are linked in the differential area

- Smear is “scanned” under 10x to review for cell distribution, any abnormal findings, smear quality.

- Smear is then “scanned” approximately 20 fields under 50x to evaluate and/or validate wbcs, and 20 fields under 100x to evaluate rbcs, and platelets as follows:

- WBC count, relative % of WBC cell types and maturity levels.

- If immature, atypical, or abnormal cells are seen, a manual differential may be added and performed. (follow manual differential procedure, supervisor review criteria, and review by clin path/attending policy)

- Visually validate accuracy of automated data vs. smear

- >10 NRBC on instrument or if NRBCs are seen on smear

- increased level of FL3 scatter on sapphire

- Note accuracy of lymph count, possible presence of NRBCs

- WBC count with persistent RRST flag

- WBC/autodiff flags, per differential protocol.

- >37.3 MCHC for spherocytes

- Evaluate RBC and platelet morphology, platelet count.

- If abnormal RBC morphology is the only significant finding, a comment is entered against RBC count.

- If platelet is only parameter requiring review, do smear estimate, if estimate and instrument agree report instrument count, if estimate and instrument count are discrepant enter smear estimate and appropriate comment(s).

- If, upon completion of the scan, the automated data is deemed accurate and no further action is required click on the “SMREV” box and choose “smear ok” from drop-down menu, verify all.  If it has been decided to add a manual diff, choose “see manual diff” from drop down box, change scan to manual diff in the Scope box, verify these fields only.

- References:

Yale New Haven Hospital Clinical Hematology Laboratory research

- History:

H-1	This procedure was written by N. Ortoli-Drew on 10-2006.

H-2	This procedure was revised by N. Ortoli-Drew on 6/2011.

H-3	This procedure was revised by S. Richardson on 6/12/2012.