| Ridgeview Medical Center Laboratory Services | Blood Bank | Effective Date: 1/11/2013 |
| --- | --- | --- |

Purpose

This procedure provides instructions for performing the FetalMaternal Bleed Screen utilizing the rosetting technique to detect Rh positive fetal cells in the circulation of an Rh negative mother.

Specimen

EDTA whole blood drawn from a post partum Rh-negative mother. Blood should be drawn at least one hour after delivery, but as soon as possible thereafter. Blood should be tested as soon as possible after collection; if a delay should occur, specimen should be stored at 1-10°C for no longer than 2 days.  Specimens are stored in the appropriate daily box in refrigerator for 2 weeks. Refer to Specimen Requirements, Labeling, and Storage.

Materials

|  |  |  |  | Supplies | Equipment |
| --- | --- | --- | --- | --- | --- |
| • Immucor FMH Rapid Screen Kit, stored at 1º-10 ºC. - Positive Control - Negative Control - Anti-D Reagent - Indicator Cells • Saline |  |  |  | • Test Tubes (12 x 75) • Pipettes • Microscope slides | • Cell washer • Microscope |
| Quality Control Positive and negative controls provided with kit must be run in parallel with each batch of tests. Testing of control cells must exhibit expected results to be valid and reportable. |  |  |  |  |  |
| Procedure |  |  |  |  |  |
| Step | Action |  |  |  |  |
| 1. | FileOpenPatient |  |  |  |  |
| 2. | MR# |  | Enter MR#  QueryOK |  |  |
| **If there is no current specimen displayed, Record Specimen and proceed to Step 9** **If there is a current specimen, the At-A-Glance-Bar will display a specimen outdate and the FMBS test will be associated with an incorrect specimen ID (pre-delivery specimen). Change Specimen ID as follows: |  |  |  |  |  |
|  | Changing the Specimen ID |  |  |  |  |
| 3. | Record the new specimen. |  |  |  |  |
| 4. | PW  √ Incomplete Work  Query (Red ?) |  |  |  |  |
| 5. | Patient testing |  | Highlight only-Do not go to RE (result entry) |  |  |
| 6. | OP on button bar |  | Click |  |  |
|  | Patient Order window |  |  |  |  |
| 7. | Specimen ID field |  | Delete accession # and scan the correct one from the specimen you are using |  |  |
| 8. | Time |  | Enter time |  |  |
| 9. | Make a 2-4% suspension of well-mixed maternal whole blood (uncentrifuged) in saline. |  |  |  |  |
| 10. | Place 1 drop of prepared cells into a properly labeled test tube. |  |  |  |  |
| 11. | Place 1 drop of positive control and 1 drop of negative control into properly labeled test tubes. |  |  |  |  |
| 12. | Add 1 drop of the Anti-D Reagent from kit to all tubes. |  |  |  |  |
| 13. | Mix well and incubate for 5 ± 1 minutes at room temperature (18° to 30° C.) |  |  |  |  |
| 14. | Wash the red blood cells four times with saline, using the cell washer. |  |  |  |  |
| 15. | Add 1 drop of Indicator Cells and mix well by gently shaking the tube. |  |  |  |  |
| 16. | Centrifuge immediately for 15 seconds at 3400 rpm. |  |  |  |  |
| 17. | Immediately resuspend the red cell button completely. Using the 10x objective, examine 5 low-power fields for mixed field agglutination. See Interpretation. |  |  |  |  |
| Interpretation Positive: Five or more clumps observed in a total of five low power fields, indicating the presence of D-positive fetal cells in possibly significant numbers in the maternal circulation. ▪ More than one dose of RhIg may be required; number determined by results of Kleihauer-Betke Negative: Four or fewer clumps observed in a total of five low power fields, indicating the absence of a large feto-maternal hemorrhage ▪ One vial of RhIg is recommended All Positive Fetal Maternal Screens  ■ order KLEI in LIS ■ whole blood specimen must be drawn and sent to reference lab for a Kleihauer-Betke ■ prepare one dose of RhIg, notify OB that Kleihauer-Betke is being sent and that one dose is ready |  |  |  |  |  |
| Criteria for FMBS and/or Kleihauer-Betke Mother; Baby; FMBS?; Kleihauer-Betke?. Rh Neg; Rh Pos; Yes; if FMBS is + Yes. Rh Neg; Weak D Pos; No; Yes. Rh Pos or Rh Neg; Unknown; No; Yes, for ante partum bleed (trauma, accident, etc.) or if ordered by the physician. Weak D Pos; Rh Pos; Invalid  test is invalid and provides no information on the extent of the bleed. The Kleihauer-Betke test detects fetal hemoglobin using an acid-elution method. Fetal cells are counted, volume of feto-maternal bleed is determined, and number of vials of Rh Immune Globulin required is calculated. Resulting Testing |
|  | Prompt | Action |  |  |  |
| 1. | PW  √ Inc. Work  Query (Red?) |  |  |  |  |
| 2. | Highlight patient testing | RE on tool bar  Click |  |  |  |
| 3. | Verify specimen field | Scan accession # on tube |  |  |  |
| 4. | Reaction Fields | Enter reactions for Positive control (M), Negative control, and patient. |  |  |  |
| 5. | Interpretation | Positive or Negative |  |  |  |
| 6. | Comment | Enter FMBS kit lot number and expiration date. |  |  |  |
| 7. | SAVE | Click and close. |  |  |  |
| RHIMM orders include G/Rh and FMBS. If only a RECK was ordered, it must be resulted manually in HLab: HLabResult Entry Enter RECK result in the G/Rh field |  |  |  |  |  |
| Ordering RhoGam |  |  |  |  |  |
| 1. | FileOpen Patient |  |  |  |  |
| 2. | FileNewOrderRegular |  |  |  |  |
| Patient Order window |  |  |  |  |  |
| 3. | Time field | T |  |  |  |
| 4. | Type field | D |  |  |  |
| 5. | Item Id field | RHIG |  |  |  |
| 6. | Quantity field | 1 (or appropriate number of vials) |  |  |  |
| 7. | Voulme field | 300 |  |  |  |
| 8. | SAVE | Click and close |  |  |  |
| 9. | Prepare RhoGam for issue: remove package insert and complete the information requested. Return insert to packaging and label with patient name. Place in Blood Bank Refrigerator until issue. (Unit tag prints at the time of issue.) |  |  |  |  |
| 10. | Call OB and notify them that RhoGam is ready to be issued. |  |  |  |  |

Issuing RhoGam

| Step | Prompt |  | Action |
| --- | --- | --- | --- |
| 1. | Patient/Order Module |  |  |
| 2. | Product (on tool bar)Issue |  |  |
| 3. | MR# field |  | Enter MR#QueryOK |
| 4. | Derivative tab |  | Click to select |
|  | Derivative to Issue window |  |  |
| 5. | Derivative ID field |  | EnterRHIG or use FIND. |
| 6. | Enter lot number from vial |  |  |
| 7. | Manufacturer |  | Enter information |
| 8. | Quantity field |  | Enter number of units issued |
| 9. | Capacity |  | Enter volume (300ug) |
| 10. | Blood Bank Tech reads and checks the following information with pickup person. Patient name and MR# (or ID Band#) on - Pink slip - Computer screen Lot Number and Expiration Date on - Vial of RhoGam - Computer screen Patient Blood Type on - Computer screen |  |  |
| 11. | Issue Location field | Enter RMC |  |
| 12. | Issue to field | After inspecting product, enter “OK” and then individual picking up product enters their first initial, last name, and title, entering “OK” ensures that the tech completed the final inspection of the product for outdate, product suitability, and packaging suitability. (Product is not contaminated/cloudy, syringe not leaking or cracked). |  |
| 13. | OK | Click |  |
| 14. | Unit Tag Now Prints |  |  |
| 15. | With Pickup Person Read and Check - Name on tag compared to pink slip - Lot number and Expiration date on tag and vial |  |  |
| 16. | Stamp the time on both copies of the unit tag and the pink slip. |  |  |
| 17. | Send tag with RhoGam |  |  |
| 18. | Place pink slip in drawer. |  |  |

Limitations

- Because the fetal screen test relies on the D antigen, this test is valid only when an Rh-negative mother delivers Rh-positive infant.

- If the mother is Rh positive, including Weak D positive, the test is invalid and provides no

information on extent of a bleed.

- If the infant is Rh negative, the test will be negative regardless of the amount of bleed

- If the infant is WeakD positive, the test may not detect a bleed of greater than 30 mls. In this case, a Kleihauer-Betke test is recommended.

- A positive DAT on the maternal red cells may cause a false positive result.

- Do not use grossly hemolyzed specimens for testing.

- In cases of ABO incompatibility between mother and child, the mother’s natural ABO antibodies may

have destroyed any fetal cells in the maternal blood circulation before testing is performed.

- Any deviation from the recommended test procedure can give false test results.

•  Incorrect specimen or use of a pre-delivery specimen.

•  Out of date reagent

•  Improper washing will cause a false positive result

•  Contaminated materials

•  Improper incubation time, temperature, centrifugation, examination for agglutination.

Safety

Refer to Laboratory Procedure #4002-1: Infection Control, Laboratory Safety Hygiene Procedure and all other safety policies for the lab.

Related Documents

Specimen Handling in Horizon Blood Bank

Resulting Testing from the Pending Worklog

Cell Suspensions and Conversions

Specimen Requirements, Labeling and Storage

Reference Consultation Services-Memorial Blood Centers

References

AABB Technical Manual, Fifteenth Edition, 2005, pages 535-538, 547-551, and 793-796.

AABB Technical Manual, Sixteenth Edition, 2008, pages 630-632.

Immucor FMH Rapid Screen package insert.

Standards for Blood Bank and Transfusion Services, 23rd Edition: 2005, pages 48-49.

| Date: |  | Author: |  |  |  |  | Date: |  |  | Validated by: |  |  |  |  | Date: |  | Approved by: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8/21/2007 |  | Mary Farlow, MT(ASCP)SBB Jo Lohmann, MT(ASCP) Cheryl Webster, MT(ASCP) |  |  |  |  | 4/1/08 |  | Cheryl Webster, MT(ASCP) |  |  |  |  |  | 6/20/2008 |  | Dr. T. Hallin, MD |  |  |
| L: Blood Bank/Fetal Maternal Bleed Rapid Screen |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Annual Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Initials: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: | Initials: |  |  | Revision and Reason |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7/1/10 | JL |  |  | Ortho kit procedure from Immucor kit procedure Dr. Weatherby, MD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3/10/11 | CW |  |  | Added procedure to delete GRH from RHIMM battery when testing already performed on current admission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10/18/11 | CW |  |  | Ordering RhoGam product in HBB to facilitate proper billing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3/28/12 | CW |  |  | Change in kit manufacturer; remove references to Weak D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1/3/2013 | CW |  |  | New incubation time and temperature; new positive and negative criteria |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Discontinued |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: |  |  | Reason: |  |  |  |  |  |  |  |  |  |  |  |  |  | Initials: |  |  |