| Ridgeview Medical Center Laboratory Services |  |  |  | Blood Bank |  | Effective Date: 9/2008 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Purpose The purpose of this procedure is to provide instructions for receipt of products from Memorial Blood Center. |  |  |  |  |  |  |  |
| Products Leukoreduced red blood cells (RBC) Platelets –random donor and apheresis (PLT) Fresh Frozen Plasma, Frozen Plasma (FFP) Cryoprecipitated AHF (CRYO) |  |  |  |  |  |  |  |
| Memorial Blood Center packs blood products in reusable validated boxes, which assure maintenance of shipping temperatures. Shipping temperatures differ slightly from storage temperatures. If products are received out of shipping temperature, notify Hospital Services. |  |  |  |  |  |  |  |
| Acceptable Temperatures Product; Storage Requirement; Shipping Requirement. Red Blood Cells; 1 o - 6 o C; 1 -10 o C. Platelets; 20 o - 24 o C; 18-26 o C. Frozen Products; -18 o C or colder; Dry ice remaining or no evidence of thawing. Procedure |
| 1. |  | When blood arrives, the temperature of red blood cells or platelets must be taken and recorded by placing the thermometer probe between the units and waiting for the temperature to stabilize. |  |  |  |  |  |
| 2. |  | Complete the received information section on the Blood and Blood Component Packing List, indicating date and time received and tech’s name, temperature on arrival and name of tech recording the temperature. If frozen product is ordered, indicate whether “dry ice is remaining”. |  |  |  |  |  |
| 3. |  | Check off units on Blood and Blood Components Packing List. Report any discrepancies or shipment temperature failures to MBC immediately by phone. In the case of a temperature failure or the product does not meet appearance guidelines, the component will be placed in quarantine until the problem is resolved or the product is disposed of. |  |  |  |  |  |
| Computer Steps |  |  |  |  |  |  |  |
| Step |  | Prompt | Action |  |  |  |  |
|  |  | Transfusion: Inventory Module |  |  |  |  |  |
| 1. |  | FILE (on tool bar) NEW | DELIVERY |  |  |  |  |
| 2. |  | Supplier drop down box | Scroll to or type MBC (Memorial Blood Center) |  |  |  |  |
| 3. |  | External No. window | Enter Ship ID # from Blood and Blood Components Packing List |  |  |  |  |
| 4. |  | OK | Click |  |  |  |  |
|  |  | Delivery window |  |  |  |  |  |
| 5. |  | Click on Type field | Enter “P” for product.  tab |  |  |  |  |
| 6. |  | Item ID field | Use Binoculars or enter product type |  |  |  |  |
| 7. |  | OK | Click |  |  |  |  |
| 8. |  | Quantity field | Enter number of units in the order from Component Packing List |  |  |  |  |
| 9. |  | Item ID field | Double-click on product type |  |  |  |  |
|  |  | Component Receipt window |  |  |  |  |  |
| 10. |  | Visually inspect the first unit |  |  |  |  |  |
| 11. |  | Visual Inspection box | Click to Select |  |  |  |  |
| 12. |  | Temp box | Enter temperature of shipment (dry ice = -30) |  |  |  |  |
| 13. |  | Scan in first unit by scanning the following barcodes in any order*: - unit number/group-Rh (may scan together) - product code/expiration (may scan together) |  |  |  |  |  |
| *MANUAL ENTRY If barcodes are damaged or unable to be scanned, units need to be brought in manually by the following procedure:. Step; Prompt; Action. 1.; MANUAL; Click. 2.; Product Code; Enter Product Code from unit label Tab. 3.; Unit #; Enter unit # including Check Sum digit  Tab. 4.; Expiration Date; Enter Julian date (0081052359)  Tab. 5.; Expiration Time; Leave field blank  Tab. 6.; Blood Type; Enter numeric blood type code. 7.; OK; Click. |
| 14. | Is temperature within range? Is appearance acceptable (not hemolyzed or abnormal in appearance)? Is packing acceptable? Yes go to Step 19 No go to Step 15 |  |  |  |  |  |  |
| 15. | - Place unit in appropriate Quarantine area of Blood Bank, freezer, or platelet agitator - Unit to remain there until resolution or disposition - Notify MBC immediately and follow their instructions - Proceed to Step 16 |  |  |  |  |  |  |
|  | Disposition field |  |  |  |  |  |  |
| 16. | Quarantine |  | Click to select |  |  |  |  |
| 17. | Reason |  | Choose from drop down box |  |  |  |  |
| 18. | Comment |  | Free text observation |  |  |  |  |
| 18a. | Platelet products enter volume from label into Avail Qty field Cryoprecipitate products Click ADDITIONAL button and enter 5 units in pool |  |  |  |  |  |  |
| 19. | Does the unit have a tag indicating antigen typing done at MBC? No go to Step 25 Yes go to Step 20 The historical antigen typing from the packing list cannot be recorded as the antigen type of record- it must be confirmed with antigen typing at RMC or MBC before being tagged as antigen negative. |  |  |  |  |  |  |
| 20. | Select ATTRIBUTES |  |  | Select FIND |  |  |  |
| 21. | Scroll to find appropriate antigen |  |  | OK |  |  |  |
| 22. | Presence flag field |  |  | Enter “-“ to indicate absence of the antigen or “+” to indicate the presence of the antigen |  |  |  |
|  | If more than one antigen has been typed  go to next step, otherwise go to Step 24 |  |  |  |  |  |  |
| 23. | ADD |  |  | Click |  |  |  |
|  | Repeat Steps 21,22,23 for each additional antigen |  |  |  |  |  |  |
| 24. | OK |  |  | Click |  |  |  |
| 25. | Is unit CMV Negative? NO go to Step 27 YESgo to Step 26 |  |  |  | Cryoprecipitate deliveries  |  |  |
| 26. | CMV Negative status barcode |  |  | Scan |  |  |  |
| 27. | ACCEPT |  |  | Click (or scan ACCEPT barcode) |  |  |  |
| 28. | Repeat Steps 13-27 for each unit in delivery |  |  |  |  |  |  |
| 29. | When all units are scanned in and accepted to lower grid  |  |  | OK |  |  |  |
| 30. | “Factors exist. Would you like to review?” |  |  | YES |  |  |  |
| 31. | Review Prohibiting Factors – “Component blood type not confirmed” |  |  | OK |  |  |  |
|  | Delivery window |  |  |  |  |  |  |
| 32. | Receipt Complete box |  |  | Select to check |  |  |  |
| 33. | SAVE icon |  |  | Click and close window |  |  |  |
| Red cell units are automatically placed on a Product Test Batch for confirmatory testing. |  |  |  |  |  |  |  |
| 34. | Place red cells in quarantine area of blood bank until tested; place Fresh Frozen Plasma in Plasma Freezer; place platelets on Platelet Agitator. |  |  |  |  |  |  |

Safety

Refer to Laboratory Procedure #4002-1: Infection Control and Laboratory Safety Hygiene Procedure and all other safety policies of the laboratory.

Related Documents Ordering Products from Supplier Returns, Credits, Discards Confirmation Testing of Donor Units Blood and Blood Components Packing List – Computerized – Memorial Blood Centers Manual Packing List – Memorial Blood Centers

References AABB Technical Manual, 15th Edition, 2005, pages 194-199. Horizon Blood Bank Quick Reference Guide. Standards of Blood Banks and Transfusion Services, 25th Edition, 2008, pages26-27, 37-38. Memorial Blood Centers Resource Guide

| Date: | Author: |  |  |  |  |  | Date: |  | Validated by: |  |  |  |  | Date: |  | Approved by: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/20/07 | Cheryl Webster, MT(ASCP) Jo Lohmann, MT (ASCP) |  |  |  |  |  | 4/10/08 |  | Cheryl Webster, MT(ASCP) |  |  |  |  | 6/20/2008 |  | Dr. T. Hallin |  |
| Receipt of Product Deliveries |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Annual Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Initials: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: | Initials: |  |  | Revision and Reason |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7/25/12 | CW |  |  | Change in temperature recording location due to HBB upgrade |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Discontinued |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: |  |  | Reason: |  |  |  |  |  |  |  |  |  | Initials: |  |  |  |  |