| Ridgeview Medical Center Laboratory Services |  |  | Blood Bank | Effective Date: 9/2008 |
| --- | --- | --- | --- | --- |
| Purpose This procedure provides instructions for receiving deliveries of reagents, derivatives, and accessories and entering necessary information into Horizon Blood Bank. Reagents must be entered into HBB and the paper log book before use. |  |  |  |  |
| Procedure |  |  |  |  |
| 1. | Reagents, derivatives, and accessories are unpacked and delivered to Blood Bank by lab assistants. |  |  |  |
| 2. | Upon receipt, blood bank tech examines all reagents and Rh Immune Globulin for abnormal appearance, cloudiness, or turbidity and places refrigerated reagents and derivatives in the “Reagents to Enter in Computer” bin on the bottom shelf of the blood bank. This is to be accomplished as soon as possible after delivery. The magnet from this bin is then placed on the white board in the “Reagent/Accessory Status” box. |  |  |  |
| 3. | Accessories are placed in the “Accessories for Computer Entry” bin. The magnet from this bin is placed on the white board in the “Reagent/Accessory Status” box. |  |  |  |
| 4. | Saline cubes are stored in the work area. A note is to be placed in the “Reagent/Accessory Status” box indicating the date received, lot number, expiration date, and quantity. |  |  |  |
| 5. | Lead tech or assistants are to record all supplies in the Reagent Log Book and perform the following as soon as possible: |  |  |  |
|  | Transfusion: Inventory Module |  |  |  |
| 6. | File(on tool bar) New  Delivery |  |  |  |
| 7. | Supplier drop down box | Scroll to correct manufacturer or type name |  |  |
|  | Delivery window |  |  |  |
| 8. | Type field | Enter G (reagent) D (derivative-RHIG) A (accessory-Component recipient set, plasma transfer set) |  |  |
| 9. | Item ID field | Use binoculars OK *If Albumin is received choose Albumin & ALB-QC *If LISS is received choose LISS & LISS-QC In Steps 10 & 12, enter identical information for both product and product-QC. |  |  |
| 10. | Quantity | Enter number received |  |  |
| 11. | Place cursor in Item ID field | Double-click |  |  |
|  | Reagent Receipt window |  |  |  |
| 12. | Required fields: - Manufacturer - Lot. No. - Expiration Date - Rec’d Directions for Use - Rev. to Package Insert? - Visual Inspection - Quantity Received | Enter manufacturer Enter lot number Enter expiration date  Y  N (unless revision sheet rec’d)  Y Enter number received Continue with Step 15 |  |  |
| Derivative Receipt window (Rh Immune Globulin) |  |  |  |  |
| 13. | Required fields: ▪ Manufacturer ▪ Lot No. ▪ Expiration Date ▪ Unit Size (g) ▪ Rec. Qty. | Enter manufacturer Enter lot number Enter expiration date Enter unit size Enter received quantity Continue with Step 15 |  |  |
| Accessory Receipt window (Component Recipient Sets, Plasma Transfer Sets) |  |  |  |  |
| 14. | Required fields: ▪ Manufacturer ▪ Lot No. ▪ Expiration Date ▪ Rec. Qty. ▪ Comment | Enter manufacturer Enter lot number Enter expiration date Enter received quantity Enter “Acceptable for Use” Continue with Step 15 |  |  |
| 15. | OK | Click |  |  |
|  | Delivery window |  |  |  |
| 16. | Receipt Complete box | Click to check |  |  |
| 17. | SAVE icon | Click and close |  |  |

Reference

Horizon Blood Bank Quick Reference Guide

| Date: |  | Author: |  |  |  |  |  | Date: |  | Validated by: |  |  |  |  | Date: |  |  | Approved by: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11/3/07 |  | Cheryl Webster, MT(ASCP) Jo Lohmann, MT(ASCP) |  |  |  |  |  | 4/10/08 7/14/08 |  | Cheryl Webster, MT(ASCP) Kristy Konerza, MLT |  |  |  |  | 6/20/2008 |  |  | Dr. T. Hallin |
| Receipt of Reagents, Derivatives, Accessories |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Annual Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Initials: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: | Initials: |  |  | Revision and Reason |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6/22/2010 | JL |  |  | Acceptable for Use entered in comment for accessories Dr. Weatherby, MD 6/24/2010 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Discontinued |  |  |
| --- | --- | --- |
| Date: | Reason: | Initials: |