| Ridgeview Medical Center Laboratory Services |  |  | Blood Bank | Effective Date: 4/9/2008 |
| --- | --- | --- | --- | --- |
| Purpose This procedure provides directions for resulting testing and for ordering and resulting Record Checks and Rechecks in Horizon Blood Bank - Record Checks: must be performed to determine whether or not the patient has a record on file. The At-A-Glance Bar displays ABO and Rh, Antibody, Comments(which may contain antibody information), Special Needs for transfusions, and transfusion reactions. - Rechecks: If the patient has no previous record, the specimen type must be rechecked by performing a front type to confirm group and Rh. See Recheck Testing procedure. |  |  |  |  |
| Procedure - Specimen must be recorded before proceeding. Orders will then go directly to the Pending Worklog. - Reactions should be read and entered directly into the computer, as work is performed. |  |  |  |  |
| Step | Prompt | Action |  |  |
| 1. | Patient/Order module |  |  |  |
| 2. | PW √ Inc. Work√ Show Electronic Crossmatch Query (Red?) |  |  |  |
| 3. | Outstanding Items grid | Highlight patient testing - To select multiple tests from the Pending Worklog, hold the <Ctrl> key down and select tests from the grid. |  |  |
| 4. | RE (on tool bar) | Select |  |  |
| 5. | Verify specimen field Specimen ID field will be the Accession # from the tube label | Scan Accession # from the tube being used for testing - If multiple patients were selected for resulting, each patient’s specimen must be verified before recording results. |  |  |
| To order the RECORD check  |  |  |  |  |
| 6. | RESULTS (on tool bar) | Add Test |  |  |
| 7. | Type RECORD or scroll to select | OK |  |  |
|  | Observe the At-A-Glance-Bar for  ABO/Rh Special Needs Antibody history Comments Transfusion Reaction history *Record checks should be resulted immediately* |  |  |  |
| 8. | Interpretation field | Enter NEW or PREV |  |  |
| 9. | Comment field | Enter date and tech initials |  |  |
| Check the At-A-Glance Bar  Is there a type on file? If not  refer to Recheck Testing procedure If there is a type  go to Step 10 |  |  |  |  |
|  | Result Testing |  |  |  |
| 10. | Result fields | Enter all test and subtest reactions (tab from field to field) |  |  |
| 11. | Interpretation field | Enter Interpretations by typing or using binoculars |  |  |
| 12. | SAVE icon | Click and close window |  |  |

Limitations If computer system is down, refer to Computer Downtime Procedure Interpretation of current tests must be compared with the patients past records to detect a possible error or potentially dangerous situation. All discrepancies are investigated and appropriate action has been taken prior to issuing a component for transfusion.

Safety Refer to Laboratory Procedure #4002-1: Infection Control and Laboratory Safety Hygiene Procedure and all other safety procedures of the laboratory.

Related Documents Computer Downtime Procedure Interpreting Reactions Recording Specimens in Horizon Blood Bank Recheck Testing

References AABB Technical Manual, 15th Edition, 2005, page 413. Standards for Blood Banks and Transfusion Services, 25th Edition, 2008, pages 44-45. Horizon Blood Bank Quick Reference Cards

| Date: | Author: |  |  |  |  | Date: |  | Validated by: |  |  |  | Date: |  | Approved by: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12/04/07 | Cheryl Webster, MT(ASCP) Jo Lohmann, MT(ASCP) |  |  |  |  | 4/9/08 |  | Cheryl Webster, MT(ASCP) Mary Farlow, MT(ASCP)SBB Jo Lohmann, MT(ASCP) |  |  |  | 12/08 |  | T. Hallin, MD |  |  |  |
| L: Blood Bank: Resulting Testing from the Pending Worklog |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Annual Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Initials: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: | Initials: |  | Revision and Reason |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3/29/12 | CW |  | Reference Recheck Testing procedure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Discontinued |  |  |
| --- | --- | --- |
| Date: | Reason: | Initials: |