| Ridgeview Medical Center Laboratory Services Code: DIFF |  |  | Department of Origin Hematology |  |  | Effective Date: 1/1993 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Purpose Peripheral blood smears for morphology of RBC's, Platelets, and WBC's shall be made in uniform "thin" preparation and appropriately stained. This procedure provides instructions for making peripheral blood smears. |  |  |  |  |  |  |  |
| Specimen Whole blood in EDTA or blood from fingerstick |  |  |  |  |  |  |  |
| Materials |  |  |  |  |  |  |  |
| Reagents |  | Supplies |  | Equipment |  |  |  |
| - Coplin jars or staining rack for Manual Staining - Wright-Giemsa Stain: Sigma-Aldrich (WG32-1L) - Buffer (Reagent A/Rinse, pH 6.8) - Distilled Water |  | - Clean glass slides (7.5 x 2.5 cm) - Diff-Safe dispensers - Glass capillary tubes |  | - small fan |  |  |  |
| Quality Control Evaluate each peripheral smear before staining; if not acceptable, make a new smear. |  |  |  |  |  |  |  |
| Procedure: |  |  |  |  |  |  |  |
| Step | Action |  |  |  |  |  |  |
|  | A specimen from a fingerstick or venipuncture is collected according to proper RMC protocol 6074. |  |  |  |  |  |  |
|  | After thoroughly mixing specimen, insert Diff-Safe device into center top of stopper in EDTA tube. Invert tube onto a clean glass slide in the midline, a short distance from one end. With Diff-Safe device resting on slide, exert pressure on tube to expel a small drop of blood. - Fingersticks: Touch a small drop of blood to a clean glass slide. Two smears should be made on all fingersticks. - Microtainer specimens: remove cap and insert capillary tube into specimen. Draw up enough to make small drop on clean slide. |  |  |  |  |  |  |
|  | Hold the specimen slide securely and lay on a flat surface. Place the smooth edge of the spreader slide on the specimen - slide just in front of the drop. |  |  |  |  |  |  |
|  | Hold the spreader slide at an angle of 30o-45o from the horizontal; draw it back against the drop of blood. |  |  |  |  |  |  |
|  | Allow the blood to spread toward the sides of the slide. |  |  |  |  |  |  |
|  | Push the spreader slide smoothly and lightly toward the opposite end, drawing the blood beneath it into a thin film. |  |  |  |  |  |  |
|  | Rapidly air-dry the blood film by using a small electric fan or fanning (waving) in the air. |  |  |  |  |  |  |
|  | Evaluate the smear according to the following: (if not acceptable make a new slide) - The blood film must have a smooth, even surface, free from ridges, waves, or holes. - These irregularities will occur if the pressure used in pushing the spreader slide is uneven. - The film should not extend to the edges or end of the slide--but should be about 3-4 cm long. - A good blood film preparation will be thick at the drop end and have a thin feather edge at the opposite end. - The thickness of the film is determined by: - The angle of the spreader slide—the greater the angle, the thicker and shorter the smear. - Size of the blood drop—a 2mm drop is sufficient. - Speed of spreading. |  |  |  |  |  |  |
|  | When dry, label the film with patient's name and date by writing with pencil on the thick end of the film or on the frosted end of a frosted slide. |  |  |  |  |  |  |
|  | The slide may then be stained with Wright's Stain, either manually or automatic slide stainer, or may be used for a special stain as desired. |  |  |  |  |  |  |

Evaluating the Peripheral Smear - More than half of the slide, or about 2 cm of the film, should be thin enough that the erythrocytes just touch each other. - Leukocytes must be evenly distributed. Evaluate the feather edge for cell distribution. - A well-stained smear will show pink to red erythrocytes, dark purple nuclei in the lymphocytes and neutrophils, a lighter purple nucleus in monocytes, bright orange granules in the eosinophils, dark blue granules in the basophils and violet to purple platelets. The cytoplasm of the monocytes will be a gray-blue. The neutrophil will have a light pink cytoplasm with lilac granules and lymphocyte will show varying shades of blue cytoplasm. - If slide and stain passes evaluation, the smear may be used for screening to confirm flagging generated by the XE-5000, for manual differentials and for peripheral morphology studies by the Pathology Department.

Procedure Notes If a MORPH (peripheral morphology) or peripherals for BONE MARROW is requested: - Make 4 peripheral smears for all Pathology Smear Review and Bone marrow peripherals. Give 2 stained smears and 2 unstained smears to the pathology department with proper paperwork. Refer to appropriate procedures for these uses.

Safety - Handle all specimens as if capable of transmitting disease. - Dispose of all biohazard materials in marked biohazard containers. - Follow RMC Policy-Laboratory Procedure #4002-1: Infection Control and Laboratory Safety Hygiene Procedure.

Related Documents/ Backup Procedure RMC Policy 6074 8511 Slide Stainer—MidasIII 8511.1 Manual Slide Staining 8516 Bone Marrow Procedure

References 1 Todd-Stanford Clinical Diagnosis and Management, Henry, 16th Ed. Vol. 1, pp. 892-894. 2 Laboratory Medicine Hematology, Miale. 3rd Ed. pp. 1127-1129.

| Date: |  | Author: |  |  |  |  | Date: |  | Validated by: |  |  |  | Date: |  |  | Approved by: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9/93 |  | Joan Goetteman |  |  |  |  | 12/09 |  | Sue Allison |  |  |  |  |  |  |  |  |  |
| /tmp/in/input.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Annual Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: | 9/94 |  | 9/95 |  | 10/96 | 8/99 |  | 6/00 |  | 11/01 | 7/02 | 9/03 |  | 7/04 | 5/05 |  | 8/06 | 8/07 |
| Initials: | jg |  | jg |  | jg | jg |  | jg |  | jg | jg | jg |  | jg | jg |  | jg | jg |
| Date: | 8/08 |  | 12/09 |  | 5/10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Initials: | jg |  | jg |  | jg |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: | Initials: |  |  | Revision and Reason |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8/99 | jg |  |  | Basofix stain |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9/03 | cw |  |  | class project/update |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1/05 | jg |  |  | Wescor instrument update |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12/09 | jg |  |  | New Stainer MidasIII/procedures separated: smear prep and staining procedure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5/10 | jg |  |  | XE-5000 update |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Discontinued |  |  |
| --- | --- | --- |
| Date: | Reason: | Initials: |