Purpose

This procedure provides instructions for issuing uncrossmatched red cells in emergency situations and for performing the post issue crossmatch.

Specimen

The specimen of choice is fresh EDTA plasma and red blood cells.

Refer to Specimen Requirements, Labeling and Storage Procedure.

Procedure:

All units issued before type and screen completed will be issued following this procedure

- Transfuse o negative red blood cells and ab plasma

- Send secureline blood-band with pickup person to be placed on patient

- Four o negative units will be issued to a single secureline blood-band

- If time permits check patient record for previous antibodies and/or other transfusion issues and notify physician immediately

| issuing an emergency tagged unit (o negative only) |  |  |  |  |
| --- | --- | --- | --- | --- |
| ******When Trauma Team is Called****** |  |  |  |  |
| Step | Action |  |  |  |
| 1. | HBBOpen Patient/Order module |  |  |  |
| 2. | ProductEmergency IssueKnown |  |  |  |
| Leave Computer Terminal Open |  |  |  |  |
| When ED calls or arrives to get blood |  |  |  |  |
| 3. | Remove unit from refrigerator |  |  |  |
| 4. | Patient button on the right side of window |  | Click |  |
| 5. | MR# field |  | Enter the Secureline Number from the tag |  |
| 6. | Query  Secureline Number will appear in bottom grid |  |  | OK |
| 7. | Issue Location: |  |  | RMC |
| 8. | Scan unit number and Standard Product Code |  |  |  |
| 9. | Place a bar-coded unit number sticker on the pink pickup slip |  |  |  |
| 10. | Retain vial with stickers and segments |  |  |  |
| 11. | Blood Bank Tech states the following, with pickup person confirming the information being read: - Secureline number on  Unit tag Unit- Pink Slip  Computer(name,Mr.#) - O Negative on  Unit tag Unit Computer - Unit Number  Unit tag UnitComputer - Crossmatch test Interpretation Crossmatch Not Performed |  |  |  |
| 12. | Visually inspect unit and check box | Visual Inspection Box - Outdate - Color - Appearance-hemolysis,aggregates, fibrin strands, clots, flocculent matter, bacterial contamination |  |  |
| 13. | Accept | Accept to lower grid. |  |  |
| additional units assigned to this secureline armband number can be scanned if needed |  |  |  |  |
| 14. | Released to: | Transport person signs: first initial, last name, title |  |  |
| 15. | Date/Time stamp both copies of unit tag, the pink slip, and the Emergency Issue Worksheet |  |  |  |
| 16. | # remind transport person to place secureline armband on patient before transfusion |  |  |  |
| 17. | Place unit in biohazard bag with armband and unit tag in side pocket. |  |  |  |
| 18. | Click OK to issue unit. |  |  |  |
| 19. | check patient record for previous antibodies and/or problems asap and notify physician if there are any issues |  |  |  |
| 20. | Fill out Emergency Issue Worksheet as completely as possible. |  |  |  |
| 21. | Complete the type and screen and the post issue crossmatch as soon as the specimen is collected. |  |  |  |
| Physician signature on unit tags is required within 24 hours of emergency issue |  |  |  |  |

POST ISSUE CROSSMATCH

FOR EMERGENCY UNITS ISSUED UNCROSSMATCHED

| If you have received a specimen and order on patient during or after emergency issue, record that specimen and record Securline number in External ID, then complete Type and Screen orders only. |  |  |
| --- | --- | --- |
| Two Twelve specimens must be received following RMC Transfer procedure. |  |  |
| Step | Action |  |
| 1. | Record specimen | Click Patient (top tool bar) Record Specimen |
| 2. | Enter Secureline number for MR No:Query Emergency issue appears at bottom of gridOK |  |
| 3. | Click on Generate button (on the right) to generate accession number |  |
| 4. | Enter Fields: | External IDSecureline Blood-Band number Phleb IDInitials Draw Date: Draw Time OK |
| Open Emergency order |  |  |
| 5. | FileOpenOrder |  |
| 6. | Enter Secureline Blood Band number for MR# QuerySelect Order (priority will be STAT)OK |  |
| Emergency Order appears RBC will be present Add XM (and if necessary a TSG1) as follows: |  |  |
| 7. | Change status from completed to ordered as follows: Orders (top tool bar) Change Status to Ordered |  |
| 8. | Prioritywill already be filled with S for STAT |  |
| 9. | Specimen ID:  Double click on field to find or click on binoculars (choose the generated accession #) |  |
| 10. | Time: Enter T for current time |  |
| 11. | Add XM orders as follows: Click Orders (top tool bar) ItemTest Battery |  |
| 12. | XM will appear on grid click OK |  |
| XM order now exists for Issued Red Blood Cells |  |  |
| 13. | SAVE and close |  |
| Complete all Testing on Pending Worklog |  |  |
| 14. | PWQuery?RE | Verify specimen ID using Generated accession# |
| 15. | Record results and interpretations following RMC procedures. |  |
| 16. | SAVE Click and Close |  |
| If incompatibility is detected at any phase of testing, the technologist must immediately notify the attending physician and the on call pathologist. Document calls in comment section of the incompatible test. |  |  |
| 17. | Cancel any extra RBC orders in HLAB |  |
| 18. | Add additional information to Emergency Issue Worksheet as completed. |  |

***Lead tech or designate will merge the two patient IDs and charge units when all testing is complete***

| 1. | To Merge patient IDs  | UtilitiesPatientMerge |
| --- | --- | --- |
| 2. | To charge for emergency units in HLab | Order entryPatient MR#order 41550 for RBCs and 41483 for XM for each unit. |
| 3. | Initial tasks as completed on the Emergency Issue Worksheet |  |

Note:

- The immediate risk of transfusing RBCs prior to pretransfusion testing being done is a hemolytic transfusion reaction caused by circulating alloantibody in the patient.

- A label with all required recipient identifiers is securely attached to the component after crossmatching and retained with the unit until the transfusion is completed.

Safety

Refer to Laboratory Procedure #4002-1: Infection Control and Laboratory Safety Hygiene Procedure and all other safety policies of the laboratory.

Related Documents

ABO Forward and Reverse, Rh-Gel Card

Crossmatch-Electronic, Immediate Spin (Gel), IgG (Gel)

Emergency Issue Worksheet

Indirect Antiglobulin Test, Gel Method

Issuing Products

Product and Specimen Transfer

Resulting Testing from the Pending Worklog

Specimen Requirements, Labeling, and Storage

References

AABB Technical Manual, Sixteenth Edition.

| Date: | Author: |  |  |  |  |  | Date: |  | Validated by: |  |  |  |  | Date: |  | Approved by: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12/29/11 | Mary Farlow MT(ASCP)SBB |  |  |  |  |  | 4/9/12 |  | Cheryl Webster, MLS(ASCP)CM |  |  |  |  |  |  |  |  |  |
| Annual Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Initials: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: |  | Initials: |  | Revision and Reason |  |  |  |  |  |  |  |  |  |  |  |  |  |  |