| Ridgeview Medical Center Laboratory Services |  |  | Blood Bank | Effective Date: 6/19/2008 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Purpose This procedure provides directions for Fya and Kell typing using the tube method. - This procedure is appropriate only if gel testing has been performed and it is determined that patient specimen reacts nonspecifically with gel. - Kell and Duffy blood group antigens are red cell antigens which are immunogenic, may show dosage, and usually are detected only by the indirect antiglobulin phase. - Antibodies to these red cell antigens are clinically significant because they may cause hemolytic transfusion reactions and hemolytic disease of the newborn. - Both Fya and Kell system antigens are well developed at birth, therefore newborn or infant testing can be performed. - Once an antibody has been identified in a patient’s serum, their red blood cells must be tested and found negative for the corresponding antigen. - Red blood cell components selected for transfusion must be negative for the corresponding antigen. |  |  |  |  |  |  |  |
| Specimen The specimen of choice is fresh EDTA red blood cells. Refer to Specimen Requirements, Labeling, and Storage. |  |  |  |  |  |  |  |
| Materials |  |  |  |  |  |  |  |
| Reagents |  | Supplies |  |  | Equipment |  |  |
| - Anti-Fya, K antisera, stored at 2-8° C. - Panel cells, stored at 2-8° C. - IgG, stored at 2-8° C - Panel Cells, stored at 2-8° C - Coombs Control Cells, stored at 2-8° C - Saline |  | - Pipet tips - 12 X 75 tubes - “Antigen Negative” unit stickers |  |  | - Cell Washer/Centrifuge - Blood Bank mirror - Pipettor |  |  |
| Quality Control Prior to routine testing, reagents must be tested with appropriate controls each day of use. Positive and negative controls must be chosen from an ABID kit: - Negative control should be a red cell lacking the appropriate antigen. - Positive control should be a red cell which exhibits heterozygous expression of that antigen. K; k; Fya; Fyb. +; +; Heterozygous; +; +. +; 0; Homozygous ; +; 0. - Testing of these control cells must exhibit expected results for patient testing to be valid and reportable. Procedure A Direct Antiglobulin Test must be performed in conjunction with the antigen typing Refer to Direct Antiglobulin Testing Procedure. Red cells which demonstrate a positive DAT cannot be accurately tested using the indirect antiglobulin test. |
| Step | Action |  |  |  |  |  |  |
| 1. | Choose appropriate panel cells for controls and prepare 4% suspension according to Procedure: Cell Suspensions and Conversions. |  |  |  |  |  |  |
| 2. | Make a 4% suspension of patient/unit cells in saline. |  |  |  |  |  |  |
| 3. | Label tubes for patient/unit, heterozygous positive and negative controls. Write antigen being tested for on tubes. |  |  |  |  |  |  |
| 4. | Add one drop of corresponding antisera to labeled tubes. |  |  |  |  |  |  |
| 5. | Add one drop of patient cell suspension to the appropriately labeled tube. |  |  |  |  |  |  |
| 6. | Add one drop of positive and negative control to the appropriately labeled tubes. |  |  |  |  |  |  |
| 7. | Mix gently and incubate for 15-30 minutes at 37°C. |  |  |  |  |  |  |
| 8. | Wash the cells 4 times with saline and decant completely after the last wash. |  |  |  |  |  |  |
| 9. | Add 2 drops IgG and mix gently. |  |  |  |  |  |  |
| 10. | Centrifuge for 15 seconds at 3400 RPM. |  |  |  |  |  |  |
| 11. | Completely resuspend cells by gentle agitation while observing for agglutination in the mirror. |  |  |  |  |  |  |
| 12. | Grade the reactions as neg, 1+, 2+, 3+, 4+. |  |  |  |  |  |  |
| 13. | Add one drop of check cells to all negative reacting tubes |  |  |  |  |  |  |
| 14. | Centrifuge for 15 seconds at 3400 RPM. |  |  |  |  |  |  |
| 15. | Completely resuspend cells by gentle agitation while observing for agglutination in the mirror. |  |  |  |  |  |  |
| 16. | Grade reactions as neg, 1+, 2+, 3+, 4+. |  |  |  |  |  |  |
|  | Note: Lack of agglutination or strength of less than 2+ in this phase indicates that either active IgG was not present or that partial neutralization of Anti IgG has occurred. In either case, the test must be repeated. |  |  |  |  |  |  |

Interpretation - Agglutination or hemolysis constitutes a positive reaction and is interpreted as antigen positive. - A smooth suspension with no agglutination or hemolysis constitutes a negative result and is interpreted as antigen negative

Result Reporting Control testing must exhibit expected results for patient testing to be valid and reportable. Refer to Antigen Testing-Quick Guide to enter results of QC, patient, and unit testing in HBB. Limitations - Red cells which demonstrate a positive direct antiglobulin test cannot be accurately tested using the indirect antiglobulin test. - Determination of patient phenotype can be difficult if the patient has been transfused within the preceding three months. - Interpretations of current testing must be compared with the patient’s past records to detect a possible error or a potentially dangerous situation. - Aged red cells may yield weaker reactions than those obtained with fresh red cells. Safety Refer to Laboratory Procedure # 4002-1: Infection Control and Laboratory Safety Hygiene Procedure and all other safety policies of the laboratory.

Related Documents Cell Suspensions and Conversions Direct Antiglobulin Testing Fya and K Antigen Typing – MTS Gel Card Interpreting Reactions Antigen Typing – Quick Flow Chart Resulting Testing from the Pending Worklog

References AABB Technical Manual, 12-14th Editions, p. 323,325 Ortho Blood Grouping Reagents, Anti-Fya, package insert, revised March 1999 Ortho Blood Grouping Reagents, Anti-K, package insert, revised May 1999 RMC Blood Bank Procedure # 8206

| Date: | Author: |  |  |  |  |  | Date: |  |  | Validated by: |  |  |  |  | Date: |  |  | Approved by: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4/20/07 | Cheryl Webster, MT(ASCP) Jo Lohmann, MT(ASCP) |  |  |  |  |  | 6/19/2008 |  | Jo Lohmann, MT(ASCP) |  |  |  |  |  | 6/20/2008 |  |  | Dr. T. Hallin |
| Fya and K Testing – Tube Method(Backup) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Annual Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Initials: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: |  | Initials: |  | Revision and Reason |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Discontinued |  |  |
| --- | --- | --- |
| Date: | Reason: | Initials: |