| Ridgeview Medical Center Laboratory Services | Blood Bank | Effective Date: 2/25/2011 |
| --- | --- | --- |

Purpose

This procedure provides directions for the performance and interpretation of crossmatches where a compatible result will not occur using appropriate testing methods, due to the presence of clinically insignificant antibodies to antigens present on donor units.

Patient antibody identification should be completed at the reference laboratory and their interpretation should indicate the proper procedure for the selection of units for compatibility testing.  If their recommendation indicates using Least Incompatible units, the following procedure should be used.

Procedure

| Step | Action |  |
| --- | --- | --- |
| 1. | Pending Worklog  RBC  PS (on the tool bar) |  |
| 2. | Select type specific units for crossmatch from the blood bank as follows: - A and O patients – select 8 units for testing - B and AB patients – select entire available supply |  |
| 3. | Perform testing using the method recommended by the reference laboratory (i.e. Gel, LISS, albumin) See appropriate testing method procedures. |  |
| 4. | Pending Worklog  XM  RE(on the tool bar) |  |
| 5. | Specimen ID Field | Scan accession # from patient’s specimen |
| 6. | Crossmatch field | Select correct testing method |
| 7. | Result field | Record reaction strengths |
| 8. | Comparing reaction strengths of all the units tested, choose the weakest reactions and result those as Least Incompatible. - If all reactions are greater than 2+, select more units and repeat the crossmatching |  |
| 9. | Once all units have been tested and evaluated, result the most incompatible units as Incompatible. |  |
| 10. | Unit tags will print for all testing resulted. - Pull Incompatible tags and discard. - Modify the tags for the Least Incompatible units by adding “Least” to the interpretation. |  |
| 11. | Place labels from unit tags on units, attach tags, and return units to blood bank refrigerator. |  |
| 12. | Units that were resulted as Incompatible and those that are not tagged for the patient, must be returned to available inventory for general use. Proceed as follows: Step; Prompt; Action. 1.; Transfusion: Inventory module Components (on tool bar) Release to Available. Release to Available window. 2.; Barcode field; Scan unit number and SPC. 3.; Accept; Click to move unit to Components to Release field. 4.; Repeat Steps 2 & #3 for all units. 5.; OK; Click and close. Units are now available for other patients.. |

| Date: |  | Author: |  |  |  |  |  | Date: |  |  | Validated by: |  |  |  | Date: |  |  | Approved by: |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2/18/11 |  | Cheryl Webster, MLSCM(ASCP) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| L: Blood Bank: Crossmatch-Electronic, Immediate Spin(Gel),IgG(Gel) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Annual Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Initials: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: | Initials: |  |  |  | Revision and Reason |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2/18/11 | CW |  |  |  | Addition of Least Incompatible Crossmatch procedure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Discontinued |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: |  |  |  | Reason: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Initials: |  |  |  |