|  | ENVIRONMENT OF CARE GUIDELINES OF PRACTICE | Number: 15.301 |
| --- | --- | --- |
|  |  | Page 1 of 3 |

HAZARDOUS MATERIALS RESPONSE CODE ORANGE

- PURPOSE

Hazardous materials received by Verde Valley Medical Center will be handled under the directives established by Standard Precautions and approved procedures for handling chemicals.

DEFINITIONS

- Container—bottle, vial, package, pack, envelope, syringe or other device devised to transport any material.

- Decontamination—the removal of all traces of potentially infectious material from a specified area.

- Spill—any hazardous material loss from containers, spills, or contamination of persons including staff, patients or visitors.

- Hazardous Material—any material which may cause injury and may be chemical, biological or radioactive material.

- Qualified Person—personnel who have knowledge and experience in management of radioactive, chemical or biological spills.

- Hazardous WasteS Officer: A person specially qualified and appointed by the VP Support And has received training in hazardous waste per Arizona Department of Environmental Quality (ADEQ) standards

- Weaponized Anthrax—bacillus anthracis spores which have been harvested and separated from cell walls. Cytoplasmic components and the surface charges are neutralized.

PROCEDURE

- Immediate Response

  - Upon receipt of any unknown substance or spill whether biological, chemical or radiological via mail, courier, or any other form of shipment, any employee calls the Hazardous Waste Officer (HWO) on a stat basis. If the HWO is not on campus, the Clinical Resource Coordinator (CRC) is notified who will then contact HWO.

  - Any employee who has contacted suspect material will:

    - Not attempt to cover, clean up or otherwise manipulate the material.

    - Contain the spill – especially if the spill may contaminate the ground water or the water system

    - Leave the room and close the door(s) immediately if appropriate

    - Remain in the immediate vicinity for evaluation by first responders.

    - Wash any contaminated skin with soap and warm water as soon as possible.

    - Avoid touching the areas on the face, nose or eyes.

  - List the affected employees by first name, last name and birth date for future reference.

  - The local ventilation system is turned off by Plant Operations. (If appropriate)

  - The affected area is identified with signage and secured.

  - The affected area is restricted to qualified person(s) trained to deal with hazardous spills and wearing personal protective equipment (PPE) applicable to the hazard.

  - The Administrator on Call (AOC) and CRC are notified on a stat basis. If AOC the HWO concurs that a significant risk has occurred, the HWO will contact the appropriate authority

  - Any chemical spill is considered to be potentially hazardous until found otherwise. Suspect biological materials (i.e.:weaponized anthrax) is considered to be potentially infectious until found to be otherwise.

  - Nuclear Medicine is called to assess a radiological spill.

  - Clothing which has been contaminated is removed and placed into plastic bags. Scrubswill be provided.

  - The Federal Bureau of Investigation would be notified of the incident by the Cottonwood Police Department (CPD) to conducts an investigation of the incident in conjunction with CPD if a criminal investigation is warranted.

- Biohazard Disinfection and Cleanup

  - Only qualified personnel perform the removal of a spill.

  - Persons engaged in removal of a spill will wear PPE appropriate for the task. See Attachment A.

  - Any sharps such as needles, broken glass, etc. are to be picked using a mechanism device, not by hand. These will be placed into a red biohazard (sharps) container.

  - If contamination is by a pathogen known to be resistant to disinfection an assay procedure approved by the Arizona Department of Health Services is utilized.

  - Waste is disposed of in red biohazard bags designed for this purpose and incinerated

- Chemical Cleanup

  - The material is identified. Obtain MSDS sheet information.

  - Qualified personnel wear appropriate PPE

  - All sources of ignition and flame are extinguished.

  - The spill is absorbed, neutralized and contained as possible. Waste is placed in yellow plastic bags and disposed according to local, state and federal guidelines.

- Nuclear (Radiological) Cleanup

Radiological spills are referred to Nuclear Medicine and the waste is disposed according to Arizona Radioactive Regulatory Agency rules.

REFERENCES:

Hazardous Waste Operations and Emergency Response, Occupational Safety and Health Administration 29 CFR 1910.120 or 29 CFR 1926.65 revised 1997.

ATTACHMENTS:  N/A or

Attachment A: Personal Protective Equipment: Removal of Infectious Agent of Unknown Origin.

Attachment B: Emergency Hazardous Materials Response Flow Sheet

| Prepared by/Title/Date: Karen Smith, Infection Preventionist , 6/13 VP Approval/Date: Rick Peterson, VP Support Services 6/13 | Committee Approval/Date: GOP Committee: 6/22/10, 6/18/13 | Dates Reviewed/Revised: 11/01,10/03, 06/06, 12/09, 5/13 |
| --- | --- | --- |