Provider Performed Microscopy Procedure

Purpose:

Provider-performed microscopy (PPM) is a testing modality that requires the use of a microscope and is performed by physicians and/or non physician practitioners during the patient’s visit. Microscopic observation of clinical specimens allows for rapid detection of the presence of bacterial, fungal, and parasitic organisms.

The formed elements suspended in the urine are precipitated by centrifugation and analyzed under the microscope Cellular elements and casts often give valuable information as to the pathology of urinary tract disease, and the detection

of metabolic or systemic diseases not directly related to the kidney.

Examination of vaginal secretions, skin, hair, and nails with a 10% KOH (Potassium Hydroxide) solution is utilized as an aid in diagnosis and classification of various yeast and fungal infections. As a strong alkali, 10% KOH digests the keratin surrounding the fungi so that the hyphae and spores can be seen.

Wet mount preparations are used to detect the presence of bacterial, fungal, or parasitic organisms, and other cellular elements indicative of pathological conditions (i.e. clue cells and polymorphonuclear leukocytes).

Examination of vaginal secretions for the presence of amniotic fluid is known as the fern test. In conjunction with patient history and vaginal fluid pH, the fern test is used to determine if the amniotic sac had ruptured. Due to the protein and sodium chloride content, amniotic fluid crystallizes to form a fern pattern when allowed to air-dry on a microscope slide.

MATERIALS:

•	Glass microscope slide and coverslip

•	Transfer pipets, disposable

•	Test tube rack

•	Centrifuge

•	Light microscope

QUALITY CONTROL:

Since commercially-prepared quality control materials are not readily available for these tests, strict adherence to the technique described in this procedure is imperative.

The microscope and lens must be kept clean at all times to facilitate the examination of the preparation. Professional microscope cleaning and maintenance services are readily available Microscope Service and Repair

(1-800717-6300).

PROFICIENCY TESTING:

The Joint Commission requires enrollment in an external proficiency testing survey designed specifically for provider-performed clinical microscopy; CLIA does not require enrollment. Proficiency testing specimens and/or photomicrographs from KOH and vaginal wet preparations are shipped twice a year and are tested in a similar manner as patient specimens. Every effort is made to integrate all proficiency testing specimens and/or photomicrographs within the routine workload and have them tested by personnel who routinely test patient specimens.

The Microscope:

Achieving an accurate result with microscopic techniques requires an understanding of the operating characteristics and limitations of the equipment used. Therefore, it is recommended that the manufacturers’ instructions are read prior to using the microscope.

I. Parts of the Microscope

1. Objective Lenses: Used to magnify the specimen a defined amount ( e.g. 10X, 40X,

100X)

2. Stage: Horizontal platform on which the specimen slide is placed

3. Condenser:

● Mounted under the stage to concentrate and focus light from the Tungsten bulb

● Raising or lowing it adjusts the amount of contrast and resolution

● Contains an aperture diaphragm, which can be opened or closed to control the

amount of light striking the specimen

4. Coarse and Fine Adjustment Knobs: Used to bring the slide into focus by moving the

stage toward or away from the objectives. The coarse adjustment moves the stage in

large increments and the fine adjustment causes subtle movement.

5. Collector Lens: Better microscopes have a field diaphragm on the collector lens. The

adjustment of this diaphragm increases or decreases the circle of light in the viewing

field

6. Tungsten Bulb: A built-in light source that can be changed by the operator when it

burns out. The light intensity can be controlled with the dimmer.

II. How to use the Microscope:

a. Place a properly labeled specimen slide (i.e. 2 patient identifiers) on the stage (2).

b. Lower the condenser and turn the lamp to low. Open both the aperature diaphragm (3)

and the field diaphragm (5).

c. With the 10X objective in place and while observing from the side of the slide and not

through the eyepiece, use the coarse adjustment knob (4) to slowly raise the stage until

the slide comes close to the objective.

d. Next, look through the eyepiece and use the coarse and/or fine adjustment until the

image is sharpest.

e. Close the field diaphragm (5) almost completely and raise the condenser until the

edges of the diaphragm are sharply focused (the condenser is usually at its highest

position). Then, open the field diaphragm slowly, stopping just as it disappears from

view.

f. Open and close the aperture diaphragm to optimize contrast. Contrast is increased by

closing the aperature. If more light is needed, turn up the lamp.

The high (40X) objective:

a. Focus and center the specimen with the 10X objective, and rotate the nosepiece slowly

to bring the 40X objective into the light path.

b. Use the FINE adjustment knob to bring the specimen into focus.

NOTE: Never raise the stage with the coarse adjustment knob when using the 40X lens.

Doing so may cause the lens to hit the slide and break.

The 100X oil immersion objective:

a. Focus and center the specimen with the 10X objective, followed by the 40X objective;

lower the stage using the coarse adjustment, with the 40X objective in the light path.

b. Place a drop of immersion oil on the coverslip of the slide, directly over the light path

and beneath the objective. Rotate the nosepiece until the 100X oil immersion objective

comes into the light path.

c. Look at the stage from the side and not through the eyepiece. Slowly raise the stage

until the objective makes contact with the oil drop. Look through the eyepiece and use

the fine adjustment knob to bring the specimen into focus

d. When finished, lower the stage and rotate the nosepiece to the 10X objective to prevent

oil from inadvertently touching the 40x objective. Clean the 100x objective with a lens

paper dampened with lens cleaner.

Maintenance

Microscope maintenance is performed annually by contract.

In addition, microscopes should be inspected, cleaned, and checked a minimum of once per week.

NOTE:

● Cover the microscope when not in use and leave the 10X objective in position

● Remove excess lotions, powders, or emollients from hands before use.

Vaginal Wet Prep

A vaginal wet mount (sometimes called a vaginal smear) is a test to find the cause of vaginitis, or inflammation of the vagina and the area around the vagina.

Specimen Collection

Collect a sampling of vaginal material with a sterile cotton swab passed through and along the area of concern. Place the swab in a clean glass test tube labeled with two patient identifiers and containing approximately 0.5 ml sterile saline. The

appearance of the saline solution prior to the addition of the specimen should be clear and colorless. If not, obtain a fresh sample. Swirl the swab vigorously in the saline to dislodge any particulates.

Testing Procedure

Perform hand hygiene and put on gloves.

1. Place a drop of patient specimen mixed with saline on a clean glass slide. Cover with

coverslip.

2. The microscope condenser must be lowered prior to reading all specimens.

3. Examine the slide carefully for epithelial cells completely covered with bacteria (Clue

cells) and for the presence of hyphae, budding yeast, or mobile trichomonads.

Microscope slides must be labeled with two patient identifiers per hospital policy.

Normal Epithelial Cells                              Clue Cell

Budding Yeast                                 Trichomonas vaginalis

Hyphae on Epithelial cells

Reporting results:

Report as presence or absence of clue cells/ hyphae or yeast/ trichomonads:

● in the patient’s record

● on a patient log or

● on another approved permanent record

Limitations:

● Non-motile trichomonads could be mistaken for white blood cells. To preserve the

motility of the organisms, the specimen should not be refrigerated but examined

immediately.

● Failure to vigorously swirl the swab in the saline to dislodge particulates may lean to

erroneous results.

● Oil droplets from intravaginal medications may be mistaken for yeast. Oil droplets

show great variation in size and are highly refractile. Yeast cells often show budding.

Quality Control Monitoring:

Traditional quality control for the Vaginal Wet Prep is not required.

KOH

10% KOH is a strong alkali used to digest cellular elements and other contaminants that would otherwise obscure the presence of fungal elements (e.g. pseudohyphae and yeast).

Specimen Collection

Vaginal specimens: Collect a sampling of vaginal material with a sterile swab passed through and along the area of concern. Place the swab in a properly labeled (i.e. 2 patient identifiers) clean glass test tube containing approximately 0.5ml sterile saline and twirl it vigorously in the saline to dislodge any particulates.

Non-vaginal specimens: Specimens should be placed immediately on a clean, glass slide. Cotton swabs should not be used to prepare the slides because cotton strands may resemble fungi.

Microscope slides must be labeled with two patient identifiers per hospital policy.

Testing Procedure

1. Perform hand hygiene and put on gloves.

2. Place one drop of specimen to a clean glass slide.

3. Add one drop of KOH solution (standard or enhanced) to the specimen.

4. Check the slide immediately for a “fishy,” amine odor and note presence or absence.

(The odor indicates anaerobic bacteria overgrowth.)

5. Allow the slide preparation to rest for up to 5 minutes for vaginal specimens and up to

30 minutes for other specimens to allow cellular tissue and other debris to dissolve.

Gentle heating may aid in clearing the mount.

6. Cover with a coverslip.

7. Scan under low power (10X) for budding yeast and pseudohyphae and under 40X

(high power) to look for smaller fungal elements.

Budding Yeast

Pseudohyphae ( w/o KOH )               Pseudohyphae with KOH

Reporting results

Report presence or absence of either yeast or hyphal elements.

Report as presence or absence of clue cells/ hyphae or yeast/ trichomonads.

Document in one of the following:

● in the patient’s record

● on a patient log or

● on another approved permanent record

Limitations

● KOH can grow fungus if stored beyond the expiration date. Do not use if solution

appears cloudy.

● Cotton strands from the swab may resemble fungal elements.

Quality Control Monitoring

Traditional quality control for the KOH test is not required.

Fern Test Examination of Amniotic Fluid

Amniotic fluid will crystallize when allowed to dry on a glass slide and will form a fern pattern. Vaginal secretions that do not contain amniotic fluid will not show a fern pattern.

Specimen Collection

1. Perform hand hygiene and put on gloves.

2. Collect vaginal secretion from the posterior vaginal pool with a sterile swab. Avoid the

use of any lubricants or antiseptics, because these may interfere with the test.

3. Do not touch the mucus plug in the cervix.

4. After collection, immediately roll the saturated swab tip across the slide while applying

slight pressure to express the fluid. Spread the specimen to create a very thin smear.

5. Allow slide to air dry for 5 to 7 minutes. Do not blow air or heat the slide to shorten

dry time. Do not apply a coverslip.

Microscope slides must be labeled with two patient identifiers per hospital policy

Testing Procedure

1. Examine the dried smear under low power (10X) without a cover slip.

2. If present, the amniotic fluid crystallizes to form a fern-like pattern due to the relative

concentrations of sodium chloride, proteins, and carbohydrates in the fluid. Examine

all fields on the slide thoroughly.

3. If uncertain about the ferning pattern, examine under high power (40/45X).

4. Dispose of slide into an appropriate sharps container.

Positive for Ferning                            Positive for Ferning

Negative for Ferning

Reporting results:

Record results in one of the following:

● in the patient’s record

● on a patient log or

● on another approved permanent record

Limitations

● The Fern Test should be performed in conjunction with a pH test. (See MGH

Phenazine pH Procedure)

● If the pH test and Fern Test are both positive, probable membrane rupture has occurred.

● If the pH test is negative but the Fern Test is positive, there is probable rupture of the

membranes due to the Fern Test's greater specificity.

● If the pH Test is positive but the Fern Test is negative, a second specimen should be

collected and tested.

● False positive results may occur from specimens contaminated with blood, urine, or

cervical mucus, which also “ferns” but usually in a coarser pattern.

●False negative results may occur from prolonged rupture of the membranes (longer than

24 hours).

● False negative results may occur if only a small volume of fluid has leaked.

Quality Control Monitoring

No commercially prepared controls are available for the fern test.

Examination of Urine Sediment

The microscopic examination of urine is the most common testing procedure used for the detection of renal and/or urinary tract disease.

Specimen Collection

Collect a FRESH urine specimen in a clean, dry container and labeled with two patient identifiers per hospital policy. Specimens that are held unrefrigerated for more than 2 hours should be rejected. Casts, RBC’s, and WBC’s are susceptible to lysis when the specific gravity is <1.010 and in alkaline urine (pH>7.0).

Test Procedure

1. Perform hand hygiene and put on gloves

2. Pour 12 mL of well-mixed urine into a properly labeled centrifuge tube.

3. Place centrifuge tube in the centrifuge and spin at 400 RCF (1500-2200 rpm) for 5

minutes. (Refer to the operator’s manual for specific RPM equivalent to 400 RCF).

NOTE: Higher RCF and longer centrifugation times, although useful in recovering cells,

will break up cellular casts.

4. Carefully decant the supernatant into a biohazard-designated sink or receptacle.

5. Gently resuspend the sediment in the remaining supernatant.

6. Using a transfer pipette transfer a drop of sediment onto a properly labeled slide, then

cover with a cover slip. Allow urine to settle for 30-60 seconds before examining.

7. Examine the urine sediment on low power for crystals, casts, and epithelial cells.

Count 10 fields and report the average number per low-power field. Lower light

intensity is required to see hyaline casts and hyphal forms.

8. Examine the sediment on high power for RBC’s, WBC’s and renal tubular cells. Count

10 fields and report the average number of cells per high-power field. Note the

presence of bacteria, yeast, trichomonads and mucus.

Elements to be reported:

WBC: None, 0-2, 3-6, 4-8, 5-10, 10-20, 20-40, 30-60, >100 Packed (per HPF). Also

report WBC clumps.

RBC: None, 0-2, 3-6, 4-8, 5-10, 10-20, 20-40, 30-60, >100 Packed (per HPF). Grossly

bloody samples should be forwarded to the Clinical Laboratory evaluation.

Epithelial cells: Few, Moderate, or Many (per HPF).

Crystals: Amorphous, uric acid, calcium oxalate, triple phosphate, tyrosine, cystine,

unidentified. In amounts: Few, Moderate, or Many (per LPF).

Bacteria: Few, Moderate, or Many (per HPF).

Casts (read under low power): Hyaline, Granular, WBC, Epithelial, RBC, Hgb, Broad,

Waxy, or Mixed.: 0-5, 5-10, 10-20, 20-50, 50-100, >100 per LPF).

Yeast: Few, Moderate, or Many.

Mucus: If present

Other: Report any additional information

Reference Ranges

WBC: Negative. Occasional (<5/hpf)

RBC: Negative. Occasional (<5/hpf)

Epithelial: Negative. Few may be present in urine from males, larger numbers in urine

from females.

Crystals: Negative. Some crystals precipitate after the sample cools. Crystals are of little

Clinical significance except for cystine, leucine, tyrosine and cholesterol. The

type of crystal depends largely upon the pH of the freshly voided urine.

Bacteria: Negative. Bacteria are of little significance except in fresh or catheterized

specimens. Gram stain and culture may be utilized for identification.

Casts: Negative.

Yeast: Negative.

Reporting results:

Record results in one of the following:

● in the patient’s record

● on a patient log or

● on another approved permanent record

Limitations:

An inaccurate reading may be caused by one or several of the following errors made in specimen collection or technique:

● Specimen not obtained by "clean catch" method and thus contains elements from

sources other than the urinary tract (e.g., vaginal discharge, penile discharge)

● Specimens that are held unrefrigerated for more than 2 hours

● Specimen not centrifuged long enough or longer than 5 minutes

● Urine extremely dilute so no sediment obtained, or not enough elements available in

amount of urine tested.

● Specimen not examined with proper lighting or focusing

● Microscope not functioning properly, e.g., lens dirty

● Examiner fails to recognize the elements on the slide

References:

1. CLSI Physician and Nonphysician Provider-Performed Microscopy Testing; Approved

Guideline-Second Edition.POCT10-A2 ; 2011

2. Wisconsin State Laboratory of Hygiene, Proficiency Test Photos, 2012

3. Addison, Lois Anne. Laboratory Medicine, July, 1999, Vol 30, No.7, p451.

4. Vincent Memorial Obstetrics Service, Fern Test Procedure

5. Bauer, John M.D. Clinical Laboratory Methods, 8th ed. C.V. Mosby Company, St.

Louis, 1974:  50-60.

6. Davstar Industries Ltd. One Newport Place, 1301 Dove Street, Suite 360, Newport

Beach, CA.

7. Todd, Sanford and Davidsohn. Clinical Diagnosis and Management by Laboratory

Methods, 16th ed. W.B.Saunders Co, Philadelphia, 1985: 559-629

8. Massachusetts General Hospital, Pathology Service 10/2012

Prepared By: Phyllis Boucher CLA(ASCP),HEW, NorDx POC Technical Consultant

Date: 06/07/2013

Effective Date: ____________

Supersedes Procedure: New

Reviewed By:___________________________Date:___________

Reviewed By:___________________________Date:__________

Reviewed By:___________________________Date:___________

Reviewed By:___________________________Date:__________

Reviewed By:___________________________Date:___________

Reviewed By:___________________________Date:__________

Approved By:______________________________________Date _______________

M.D., Medical Director