UW Medicine - Pathology

6000-01-03-05

Neuropathology Specimen Accessioning Procedure

Adopted Date: 09/08/05 Review Date: 06/22/09 Revision Date: 04/13/11

PURPOSE

To provide instructions for handling and transporting neuropathology tissue specimens to the neuropathology department at Harborview Medical Center for processing.

PROCEDURE

Materials:

- Gloves

- Lab Coat or Vinyl apron and sleeve protector

- Eye protection

- Specimen

- PCR Requisition

- Power Path Application

- Scanner

- Specimens generated by neurosurgeons should be accessioned, placed in specimen containers and fixed in formalin. If the specimen is a muscle or nerve biopsy please refer to those respective protocols.

- All neuropathology cases are to be accessioned with a NP prefix.  The case type will default to H NP Surg.  The case type must be changed to UW NP Surg.

- The case should be assigned to the neuropathology fellow.  Please refer to the posted resident rotation schedule.

- The fee schedule field should read UWMC Fee, since the procedure was performed at UWMC.

- The case priority is always routine.  For nerve and muscle specimens refer to Transportation of Neuropathology Specimens procedure.

- Under REQ DATA tab, subcategory General Information:

  - Case = Surgical

  - Accessioned at = UWMC

  - Order Station # = 850

- Specimens are all listed under the NP heading.

- Fix all fresh specimens with Formalin in appropriate containers, and label requisition and specimen containers with NP #.

- Place specimens in designated holding area in the main gross room.

- Every weekday at 2:00 pm, enter (book, including date, accession #, patient name and specimens) into NP log book.

- Place all specimens and requisitions in transport box with the label reading Hold for HMC, and bring to Histology.

- Call HMC, 731-3910, and inform them they will be receiving specimens.

REFERENCE

Transportation of Neuropathology Specimens procedure, 6000-01-06-01

Written By:					Director Approval:

(Signature and Date)				(Signature and Date)

_______________________		_________________________