UW Medicine - Pathology

100-05-01-13

Universal Precautions

Adopted Date: 07/25/95 Review Date: 01/27/09 Revision Date: 05/20/11

PURPOSE

To ensure that all specimens be treated using universal precautions in order to prevent the spread of infection (e.g. tuberculosis, etc.).

SCOPE

All department staff, residents/fellows, and faculty.

POLICY

Gross Room / Frozen Section Room:

The Gross Room and Frozen Section room are to follow all universal precautions for all specimens.  If information regarding airborne / bloodborne contamination is received, the individual handling the specimen should take extra precaution during examination / processing to avoid infection.  All equipment and surfaces should be immediately decontaminated upon completion of process.

Cytology - Non Gyn

- All unfixed non-gyn specimens are to be processed inside a Biosafety cabinet.  Never open the specimen containers until the fan is activated.  Always work as far back in the cabinet as possible.

- Dust mist masks are used when "blending" specimens under the hood to minimize the possibility of contamination with aerosols.  Safety goggles are to be worn for eye protection.

- Specimens and equipment, such as blender containers, are decontaminated or contained before removal from the hood.

- Allow the hood to operate at least ten minutes after completing all operations.

- Cover the work surface with a pad soaked in disinfectant when processing specimens.

- Cytocentrifuge rotors are to be opened within the hood.

Autopsy

Refer to the Safety, Infection Control Autopsy Procedure, 5000-09-01-01, in the Autopsy manual.

For a list of universal precautions refer to the Safety, Infection Control Management Policy, 100-05-01-01.

REFERENCE

Safety, Infection Control Management Policy, 100-05-01-01

Safety, Infection Control Autopsy Procedure, 5000-09-01-01

UWMC Pathology Chief of Service:	HMC Pathology Chief of Service:

(Signature and Date)				(Signature and Date)

5/24/13	5/24/13

____________________________		___________________________

Suzanne Dintzis, MD, PhD			Stephen Schmechel, MD, PhD

Written by:					Revised by:

(Signature and Date)				(Signature and Date)

_________________________		_________________________