## TITLE:  QLn Series Cleaning

## Principle: Keep printers clean and able to read print

Personnel: Phlebotomists

Stepwise Procedure:

| Area | Method | Interval |
| --- | --- | --- |
| Printhead (Figure 24) | Use the supplied cleaning pen or 70% Isopropyl alcohol on a cotton swab to clean the print elements from end to end (the print elements are located in the thin gray line on the printhead). | After every five rolls of media (or more often, if needed), Linerless media requires more frequent cleaning |
| Platen Roller (Figure 24) | Rotate the platen roller and clean it thoroughly with the cleaning pen or 70% Isopropyl alcohol and a cotton swab. |  |
| Platen Roller (Figure 25) | Units with linerless platens: Rotate platen & clean bearing points only. Avoid use of alcohol on the surface of linerless platens |  |
| Peel bar (Figure 24) | Clean thoroughly with the cleaning pen or 70% Isopropyl alcohol and a cotton swab. | As needed |
| Tear bar (Figure 24) |  | As needed |
|  |  | As needed |
| Exterior | Water-dampened cloth or 70% Isopropyl alcohol wipe. |  |
| Interior (Figure 24) | Brush/air blow. Ensure the Bar Sensor, Gap Sensor and Label Present Sensor windows are free of dust. |  |
| Interior (Figure 25) | Units with linerless platens: Clean inside surfaces of edge guides & media rollers with the supplied cleaning pen or 70% Isopropyl alcohol on a cotton swab. | After every five rolls of media (or more often, if needed) |