## TITLE:    Critical Values - Coagulation

## PRINCIPLE:

To alert the physician when test results exceed a critical limit.

### PERSONNEL

Medical Technologists and Technicians

### REAGENT PREPARATION & EQUIPMENT

See individual test procedure

SPECIMEN COLLECTION/TREATMENT:

See individual test procedure

### QUALITY CONTROL

Run per procedure for test.  It is recommended that Quality Control be run with the critical value sample to verify results are not instrument or reagent error.

### STEPWISE PROCEDURE

1.	The following are critical values:

PT/INR:	INR: Greater than or equal to 4.0

PT greater than 130 seconds

No Clot Detected

PTT:	           Greater than 75 seconds

No Clot Detected

2.	Rerun and/or verify critical values.

3.	Report results through the LIS system

4.	 All critical results are called and documented per the RBTO (read back) protocol.

5.	The Medical Technologist or Technician calls the results to the nurse or physician in charge of the patient.  Identify to the nurse or physician the critical result.  Enter into the result message the full name of the nurse/physician and the critical test name. Your initials and the date and time auto fill by the system.