## TITLE:  Disposal of Laboratory Waste

### PRINCIPLE:

Laboratory waste consists of chemical, biological and regular (paper) waste.  Various city, state and federal regulations govern the disposal of laboratory wastes.  The hospital uses a licensed waste hauler who provides manifests of what is taken and returns documentation of destruction.

PERSONNEL:

###### All Laboratory Staff

### STEPWISE PROCEDURE:

- Biohazards

A.	Any blood specimens, body fluids, materials grossly contaminated with blood, anatomical specimens, microbiology culture media, and sharps are disposed of in puncture and leak proof biohazard containers or cardboard biohazard containers lined with 2 reinforced biohazard bags.  These containers are closed when they are 2/3 full and removed by Environmental Services (EVS).  EVS will store the biohazard container in the hazardous waste holding area for pick up by a contracted waste removal vendor. The biohazard waste is removed from the premises by a licensed waste hauler to be incinerated per governmental protocols.

B.	Placentas are delivered to and disposed of by the histology department. Placentas are stored for diagnostic retrieval for a designated period of time.  At the end of this period, the formalin is neutralized and discarded.  The placentas are placed into double, biohazard bags and placed into puncture resistant biohazard containers.   EVS will store these containers in the hazardous waste holding area for pick up by a contracted waste removal vendor.

2.   Chemical Waste

A.       Any carcinogen must be disposed in an approved regulatory site by an approved chemical waste vendor.

B.        Large amounts of solid chemical waste must be removed by an approved chemical waste vendor.

C.        Small amounts of volatile liquid wastes (less than 1 mL) can be dumped down the drain and flushed with large volumes of water.  Greater than 1 mL must be sent to an approved regulatory site by an approved chemical waste vendor.

D.       Chemical spill residue is to be disposed of into a special waste receptacle.

The residue will be placed in landfill by contracted services.

E.      All chemicals are inventoried annually and purged as necessary.

F.      Laboratory chemical waste removed from Copley by an                                                                                                                           approved vendor requires a documented Hazardous Waste manifest.

G.       Used Xylene is placed in a waste bottle, labeled/dated and                                  	           kept in the flammable cabinet until disposal.  To dispose, the waste

bottles are taken to the 55 gallon drum housed in flammable

cabinet on the loading dock.   Wasted xylene is added to the drum

using gloves, and goggles.  The drum is labeled with the contents and the     date on which the first waste was added.

H.       10% formalin is widely used in the histology lab.  All formalin is neutralized

with a formalin neutralizing agent prior to disposal. The neutralization                                 process renders the chemical inert and non-hazardous and is disposed of in the regular trash.

I.         Alcohol waste is flushed down the sink with large amounts of running   water.

J.        Acid that needs to be discarded is taken to a storage drum on the loading dock and removed by a licensed waste hauler.

.

- Syringes, needles and sharps waste.

- Syringes and needles disposed of in plastic puncture resistant biohazard     containers.

- Sharps must be disposed of either in 1 gallon plastic or cardboard container lined with a red biohazard bag.

- When the 1 quart or 1 gallon plastic biohazard containers are 2/3 full, they are closed and picked up by EVS who take them to the biohazardous holding area for pick up by a contracted waste removal vendor.

- Cardboard biohazard containers which are 2/3 full, have the biohazard bags tied off without removing them from their original containers and are closed and taped. EVS will store in the hazardous waste holding area for pick up by  a contracted waste removal vendor.

4     Fluids

- Small volume fluids are kept in the original container, tightly sealed, and placed in the lab biohazard waste containers.

- Large volume fluids in Histology (such as pleural fluids) are poured down the Histology drain (with garbage disposal) flushing with large volumes of water.

- Fluids received in large unbreakable, unopened bottles can also be disposed of in the lab biohazard waste containers.