## TITLE:  Accumulation of Normal Population Statistics Protocol

## PRINCIPLE:

“Normal” reference ranges very for each population that is tested; therefore’ it is essential to establish normal reference ranges for each test that is run in the Chemistry department.

# SPECIMEN

Patient Preparation – No special preparation required for general chemistries.

- Lipid profiles require a fasting specimen

Type of Specimen –Lithium Heparin, SST or Urine depending on Analyte

Handling Conditions – Handle all specimens as infectious material.

# CALIBRATION

No standard preparation required.

# QUALITY CONTROL

Refer to the quality control procedures:

### STEPWISE PROCEDURE

- General Chemistries

- Choose 25 Male and 25 Female fasting specimens that have been drawn in the correct tubes.

- Run the assays on all 50 specimens on the chemistry analyzer.

- The results will then be calculated for the mean and SD using the EP Evaluator program.

- The mean and +/- 2 SD will be established.

- Record results.

- If the new mean correlates with the current normal values, accept the data.

- If the new mean, does not correlate with the current normal values, contact the Chemistry Senior Technologist or designee so that they can adjust the ranges.

- Urines

- Choose 50 random urines from the Urinalysis department.

- Run 50 urine specimens on the Osmometer and on the chemistry analyzer. Run Na+, K+, Cl-, Amy, UPro, Crea, Phos, BUN, uAlb

- The results will then be calculated for the mean and SD using the EP Evaluator program.

- The mean and +/- 2 SD will then be established.

- Record results.

- If the new mean correlates with the current normal values, accept the data.

- If the new mean, does not correlate with the current normal values, contact the Chemistry Senior Technologist or designee so that they can adjust the ranges.

# REPORTING RESULTS

None