## TITLE:  Signing in Units of Blood from RCMC to RCHC

PRINCIPLE

Blood should never be left unnecessarily at room temperature.  Delayed deliveries to the ECY, delayed arrival of equipment or infusion are all undesirable.  Blood stored in the Yorkville Laboratory have refrigerators that meet American Association of Blood Bank's specifictions.  Donor blood must never be stored in unmonitored refrigerators.

### PERSONNEL

Medical Technologists

### STEPWISE PROCEDURE

- Blood may be signed back into RCHC Yorkville Lab if it has not been allowed to warm above 9.5°C as indicated by the nonreversible Safe-T-Vue 10 indicators.  Blood will arrive hand delivered.

- Check the Safe-T-Vue indicator.

| All White | < 9˚C | Okay to use | The blood product temperature was maintained within proper transport temperature. |
| --- | --- | --- | --- |
| Random Red Spots | ≈ 9.5 ˚C | Okay to use | The blood product should be cooled or re-refrigerated immediately. |
| All Red | ≥ 10˚C | NOT okay to use | The blood product temperature exceeds the allowable transport temperature, the blood product must be quarantined and/or discarded. |

- Label should indicate a temperature 9.5°C or below.  Blood is then returned to the refrigerator for reissue.

- If blood has warmed above 9.5°C (ALL RED), the technologist should notify the supervisor                      explaining the circumstances responsible and quarantine the unit, see Proc. #7180-ECY-402.

- The person accepting the returned blood will enter the product using In SoftBank II 25.3.0.3.15-Site Y-LIVE launch “Inventory”, “In/Out” and “Batch Delivary”

- Source: MIN (Rush-Copley Medical Center)

- Coll. Ctr.: Heartland Blood Center (*0397)

- Temp: 4

- Box opens to scan multiple units

- When scanning units a box opens multiple times with comment “The line will be cleared. Accept?” say NO to keep the information scanned in.

- After units scanned place coursor on an empty line.

- Select F12

- Batch unit delivery edit box opens, F12 to accept information.

- “Accept unit list?” pop up box opens, select Yes.

- Make copies of the units and attach the printout to each unit.

- An Emergency Tag for each unit with the Unit #, Product, Exp. Date, will be already attached from the Main Lab.

- Place units in blood storage refrigerator top shelf in the Yorkville Lab if acceptable to use.

- If LIS is down then use LIS Blood Downtime Log (see example) to document the units that have been received at RCHC.

- Qurantined units are placed on the second shelf of the Blood Storage refrigerator.

- These units are disposed of in accordance with hospital policy

4840-SAFT-100, Laboratory Biosafety Procedures for Level 2 Clinical Laboratory.

### INTERPRETATION

Blood which has been returned to the Yorkville Lab shall not be reissued unless the following conditions have been observed:

- The container closure has not been disturbed.

- The blood has not been allowed to warm above 9.5°C storage or transportation.

- The records indicate that the blood has been reissued, and that it has been

inspected prior to reissue.

- Products must be checked for expiration date and abnormal appearance, such as color, hemolysis, clots, and bag intergrity.  Comparison of bag and segment color should be performed for red blood cell units as an aid in detecting bacterially-contaminated units.

### REFERENCE

Biosynergy, Inc.

AABB Technical Manual,16th Edition, 2008.

Standards for blood Banks & Transfusion Service 25th Edition, 2008.

SCC Soft Computer, 5400 Tech Data Drive Clearwater, Florida 33760  USA