Purpose:

To provide instructions for using the Blood Product Issue (BPI) function in Sunquest to issue allocated blood products for Transfusion.

Method:

|  | Actions | Computer Processes |
| --- | --- | --- |
| 1 | Receive request for component to be sent. | Open Blood Product Issue (BPI). |
| 2 | Using the Patient HID to search | - Enter patient HID. Confirm patient information from component request to the patient information displayed in the screen header. The cursor will be at component type field. Enter component type. Select “add” or alt A and then “select” or alt S. - While you are issuing units, you can view information about linked patients, autologous and directed units, and view other laboratory results for the patient. - If necessary, you can modify the shift and tech information before selecting a patient, and specify that you want to issue in emergency mode. - Select the units by scanning the unit number or selecting the check boxes. To select all units, click Select All. Continue. - Cursor is at visual inspection. After checking unit and paperwork as per Blood Product Issuing Policy select “pass all” then “continue”. - Tab through entries for current date and time or enter different time if not issuing real time. Enter Issued To box with ________ (Name of person receiving the product). Save. Under Unit comment type a semi colon, then enter how the unit left (ie. Tube, TS for someone coming to pick up from us, ER, or OR.) You must type a semi colon first to get a text box. - Additional billing box will come up. Select cancel and issue is complete. |

|  | Actions | Computer Processes |
| --- | --- | --- |
| 2.1 | Using the unit bar code to search | - Select look up by unit number then scan unit. - If only one accession number exists, computer will default to next screen. If more exists, choose the associated accession number with unit to issue. - Confirm patient information from component request to the patient information displayed in the screen header and on the request paperwork. - Select the proper circle on “visual inspection ok” radio dial. If YES proceed to next step. If NO then resolve the issue and/or select another unit if allocated. - Tab through entries for current date and time or enter different time if not issuing real time. Enter Issued To box with ________ (Name of person receiving the product). Save. Under Unit comment type a semi colon, then enter how the unit left (ie. Tube, TS for someone coming to pick up from us, ER, or OR.) You must type a semi colon first to get a text box. - An additional billing box will appear for the unit issued. Select cancel unless instructed otherwise and issue is complete. |
| 3.0 | Branch from Blood Order Processing to Issue Product | - Select all or check boxes of single unit(s) to issue. Continue. - Cursor is at visual inspection. After checking unit and request paperwork select “pass all” then “continue”. - Tab through entries for current date and time or enter different time if not issuing real time. Enter Issued To box with ________ (Name of person receiving the product). Save. Under Unit comment type a semi colon, then enter how the unit left (ie. Tube, TS for someone coming to pick up from us, ER, or OR.) You must type a semi colon first to get a text box. - Additional billing box will come up. Select cancel and issue is complete. |
| 4.0 | Visual inspection is not okay – need for quarantine or discard | - If you are issuing and at visual inspection notice that the unit is not okay for issue, then instead of selecting the pass all key, select Inspect Unit. This will lead to another screen where your visual inspection defaults to No for not okay and allows you to quarantine or discard the unit and select a reason code with additional free text if necessary. All yellow fields are mandatory entries. |

References:

AABB Standards for Blood Banks and Transfusion Services, Current Edition

Blood Bank User’s Guide, Misys Laboratory