UW Medicine - Pathology

6000-02-02-06

Bone Marrow Biopsies Microtomy Procedure

Adopted Date: 08/01/08 Review Date: 12/14/10 Revision Date: 06/13/11

PURPOSE

Instructions for cutting Bone Marrow Biopsy slides:  Parts B (the decalcified Bone Marrow Biopsy in an orange cassette) and C (the Particle Prep in a white cassette) are typically processed and embedded for cutting and staining in Histology.  All Bone Marrow (LM) blocks should be cut and stained to meet the 8:00am delivery deadline.

SCOPE

Histology Department

PROCEDURE

- The Bone Marrow Biopsy should be cut at 4 microns and placed on charged slides as follows:

- 8 levels will be cut onto 8 slides - Check the special stains work list for any additional specials that may have been ordered, and cut those at level 3.

- Levels 1, 3, and 5-8 should be unstained slides with 1 section per slide, mounted on the lower half, baked in a 60OC oven for 30 minutes, and given to IHC. IHC will file these slides for future staining.

- Levels 2 and 4 should be H&E stained with 2 sections per slide, labeled and scanned for delivery.

- The unstained slides should be marked as completed in the computer.

  - The Particle Prep (Part C in a white block) should be cut at 4 microns and only 1 section placed on a charged slide for H&E staining.

All Bone Marrow (LM) H&E slide should be cut and stained to meet the 8:00am delivery deadline.  The delivery location is a drawer near the hallway door in Histology.

Written By:					Director Approval:

_______________________		___________________________

Histology Supervisor