UW Medicine - Pathology

6000-02-07-01

Microscope QC / Maintenance Procedure

Adopted Date:2/18/2011 Review Date: Revision Date:

PURPOSE

This procedure has been established to ensure that equipment utilized in the technical aspects of patient care is regulated properly.  It has been implemented to ensure that the highest quality technical services are rendered to the patients and physicians at University of Washington.

PROCEDURE

Materials:

- Microscope

- Lens Paper

- Gauze

- Xylene

Procedure:

The Microscope is on a yearly PM service performed by Scientific Instruments.

- Clean the lenses frequently with lens paper.  Do not use other paper tissue.

- If immersion oil is used, immediately remove after use.

- Use xylene on the objectives only as a last resort, and then use it sparingly and remove it immediately.

- Clean the slide stage of residual cover-slipping glue.

- Turn the light off when Microscope is not in use.

- Remove slides from the stage when the Microscope is not in use.

- Do not touch the lens surfaces with the fingers.

- Clean light filters with lens paper as needed.

- Notify supervisor of any discrepancies.

- If service is needed, call Scientific Instruments at: 206- 543-5580

REFERENCES

Histotechnology, a Self-Instructional Text, pg 44-45 Freida L. Carson

Written By:					Director Approval:

_______________________		___________________________

Histology Supervisor