UW Medicine - Pathology

2000-06-01-11

Refrigerators and Freezers QC Procedure

Adopted Date: 02/09/93 Review Date: Revision Date: 04/08/13

PURPOSE

To monitor quality control of all the refrigerators and freezers.

PROCEDURE

- Daily -

Temperature will be monitored thru the Temp Track monitoring system. Refer to the Temp Track Procedure (2000-06-0-15)

- Other -

  - Refrigeration Shop performs preventative maintenance on the unit every 12 months.

HR-2 leak test system, repair all leaks

HR-3 change compressor oil and oil filter elements

HR-4 change refrigerant filters

HR-5 check safety controls, record settings

HR-6 check operating controls, record settings

HR-7 check electrical connections

HR-15 Inspect condenser coil, clean as needed

HR-22 Record running amps and volts

  - For problems, call Operation and Maintenance at 598-4911 or the refrigeration Shop at 598-3146.

RELATED DOCUMENT

Temp Track Procedure.

Revised By:					Director Approval:

(Signature and Date)				(Signature and Date)

_______________________		___________________________