UW Medicine - Pathology

6000-01-01-07

Therma Shandon Microwriter Cassette Printer Procedure

Adopted Date: 07/25/02 Review Date: 04/15/11 Revision Date: 04/30/07

PURPOSE

To instruct the proper use the Therma Shandon Microwriter Cassette Printer..

PROCEDURE

- Check format field (upper right): should be on Surg 01

- Check hopper, select for correct color of cassette (white for surgicals, yellows for biopsies).

- Set filed in cassette message area:

  - SU if normal, UF if Autopsy, NP if Neuropath, etc.

  - Click on case number: range = 1, type correct number (changing range will also print next number)

  - Click on specimen letter designation, type in correct letter: range = 1, for next letter range = 2 (1=a, 2=b, 3=c, etc)

  - Year is set, this cannot be changed.

  - Number of cassettes, type in number of where printing should begin: range = number of cassettes to be printed.

    - When all fields selected, click on "send to print queue"

    - To print, click "start printing"

    - To pause, click "pause printing"

Written By:					Director Approval:

_______________________		___________________________