Purpose

To provide instruction for performing component preparation of thawed plasma in Sunquest, using the Blood Component Preparation (BCP) function.

Method

| Step |  | Actions |  | Related Documents |
| --- | --- | --- | --- | --- |
| 1 |  | Thawing frozen plasma - Thawing is performed in the Helmer Quickthaw System - Inspect the thawed product to assure it is intact and passes visual inspection. |  | - Thawing Products Using the Helmer Quickthaw System - Visual Inspection of Plasma Products |
| 2 |  | Component Preparation in SQ - Open BCP - Select appropriate code or use search feature to select from the following options: - THDIV Thaw 5 day divided plasma Note: Divided plasma will have the letters AO, BO, CO, or DO as part of the ISBT code, which should have scanned in at BPE. - THFFP Thaw FFP for single draw plasma frozen within 8 hrs - TH24H Thaw FP for single draw plasma frozen within 24 hrs. - THCRYR Thaw 24 hr cryo-reduced plasma - Tab through date and time if doing processing real time. Adjust date and time if necessary. - Accept default shift and Tech ID or change if needed. Continue. - Scan in Unit Number - Acknowledge QA Warning that test BLC has not been performed. - The window on the right of the screen displays the data for the product selected from the window on the left of the screen - Enter any missing mandatory (yellow fields are mandatory) data for each output and new unit. - The lower screen shows the Task Summary and each unit will have detailed Input and Output data. - Review the Output new expiration date and time for accuracy. (See Table A) - Task Summary list can be printed if desired by selecting Task Summary on bottom left of screen. |  | - Blood Label Check (BLC) and Verification - Label Verification Form |
| Step | Actions |  | Related Documents |  |
|  | Component Preparation in SQ continued. - Select SAVE. The system will ask if it should file all units. - Select OK. The Output/New Units window opens, showing the results of the component preparation. - Select CLOSE - A Hematrax Label will print. Remove the label from the printer. - Open BLC and perform Blood Label Check - For divided plasma, record ACD volume on the Label Verification Form and verify with a second tech. - The label must be printed and verified. - The product will not be in available status for issue until BLC is resulted. |  | - Blood Label Check (BLC) and Verification - Label Verification Form |  |

TABLE  A          Expiration Dates

| Product Type | Expiration on Label at Draw | Expiration on Label Post Thaw |
| --- | --- | --- |
| FP (F24 or FFP) | 12 months from date of collection, if stored at -18C | 5 days from date of Thaw or same as original label, whichever is soonest. |
| Cryo or Pooled Cryo | 12 months from date of collection | 4 hours from time of Thaw |
| Apheresis Platelets, Combined | 5 days from date of collection | 4 hours after combination |
| Cryo Reduced Plasma | 12 months from date of collection | 24 hours from time of Thaw |

References:

Blood Bank User Guide, Misys Laboratory

AABB Standards for Blood Banks and Transfusion Services, Current Edition.